# EMC

Symmetrix Open Replicator Migration -- steps

ssunacs6

Host WWNs

10000000c93dece0  #1012-7aA #1153-4bA  #2246-8cA

10000000c9436380  #1012-10aA #1153-13bA  #2246-9cA

1. Identify Target frame/FAs.

2. Get the details of source devices and cut the disks in target frame with same properties(meta config/size) and map to

decided FAs.

3. Get the zoning done for SRC frame FAs(to which the devices are mapped) with TGT frame FAs. (Done)

#1012 FA-7aA 500604844a379106 with 2246 FA-8cA 5006048c52a6f987

#1012 FA-10aA 500604844a379109 with 2246 FA-9cA 5006048c52a6f988

#1153 FA-4bA 5006048452a5e853 with 2246 FA-8cA 5006048c52a6f987

#1153 FA-13bA 5006048452a5e85c with 2246 FA-9cA 5006048c52a6f988

4. Mask the source devices to Target FAs. (Not yet done)

#1012 devices to #2246 8cA/9cA

pbrun symmask -sid 1012 -wwn 5006048c52a6f987 -dir 7a -p 0 add dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 5006048c52a6f988 -dir 10a -p 0 add dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 devices to #2246 8cA/9cA

pbrun symmask -sid 1153 -wwn 5006048c52a6f987 -dir 4b -p 0 add dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

pbrun symmask -sid 1153 -wwn 5006048c52a6f988 -dir 13b -p 0 add dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

5. Mask the TGT devices to Host

pbrun symmask -sid 2246 -wwn 10000000c93dece0  -dir 8c -p 0 add dev 5BC,238,3BE,3C3,3C8,3CD,41A,41F,424,429,42E,433,438,43D,442,45E,463,28D,290,4C6,2BC,468,46D,472,2C9,2F7,368,371,37A,234,4C8,477,47C,5BD,4CA,4CF,5C3,5C9,5CF

pbrun symmask -sid 2246 -wwn 10000000c9436380   -dir 9c -p 0 add dev 5BC,238,3BE,3C3,3C8,3CD,41A,41F,424,429,42E,433,438,43D,442,45E,463,28D,290,4C6,2BC,468,46D,472,2C9,2F7,368,371,37A,234,4C8,477,47C,5BD,4CA,4CF,5C3,5C9,5CF

6. Get the confirmation from Platform team regarding newly added devices.

7. Stop the Application/Bring down the database/Shutdown the server.Get confirmation from platform team.

8. Suspend the SRDF/S for the Source devices to maintain point-in-time copy on the R2 (CT2) site.

9. Create the DGs for SRC/TGT devices to split/establish easily.

pbrun symdg create ssunacs6_1012 -type rdf1

pbrun symld -sid 1012 -g ssunacs6_1012 add dev < 117 & remaining devices also>

## Split the Sync pair of SRC devices

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 que

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 suspend

pbrun symdg create ssunacs6_1153 -type rdf1

pbrun symld -sid 1153 -g ssunacs6_1153 add dev < 181 & remaining devices also>

## Split the Sync pair of SRC devices

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 que

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 suspend

pbrun symdg create ssunacs6_2246 -type rdf1

pbrun symld -sid 2246 -g ssunacs6_2246 add dev < 5BC & remaining devices also>

## Split the Sync pair of TGT devices (If already pair is created for new devices)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 que

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 suspend

## Split the Async pair of TGT devices (If already pair is created for new devices)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 que

(We need to follow Async steps like changing mode of rdfg to acp ..)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 suspend

10. Un-Mask the source devices from Host.

#1012 Source devices from host

pbrun symmask -sid 1012 -wwn 10000000c93dece0   -dir 7a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 10000000c9436380   -dir 10a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 Source devices from host

pbrun symmask -sid 1153 -wwn 10000000c93dece0 -dir 4b -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1153 -wwn 10000000c9436380 -dir 13b -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

11. Make the Source devices "Write_Disable"  (Pls. be careful when executing this command.Crossverify Frame ID and devices)

pbrun symdev -sid 1012 write_disable -file ssunacs6_1012_srcdevs -nop

##ssunacs6_1012_srcdevs (create all these files under 22june2008)

0117

0118

011D

0122

0127

012C

0131

0136

013B

0140

0145

014A

014F

0154

0159

015E

0163

0168

016B

016E

pbrun symdev -sid 1153 write_disable -file ssunacs6_1153_srcdevs -nop

##ssunacs6_1153_srcdevs (create all these files under 22june2008)

0181

0189

025C

0261

02CA

02D2

02DA

02E2

02EA

0524

0529

052B

0530

061C

06A6

06AB

072F

0735

0826

12. Make the target devices "Not_Ready" (Pls. be careful when executing this command.Crossverify Frame ID and devices)

pbrun symdev -sid 2246 not_ready -file ssunacs6_2246_tgtdevs -nop

5BC

238

3BE

3C3

3C8

3CD

41A

41F

424

429

42E

433

438

43D

442

45E

463

28D

290

4C6

2BC

468

46D

472

2C9

2F7

368

371

37A

234

4C8

477

47C

5BD

4CA

4CF

5C3

5C9

5CF

13. Create Cold-Pull session between SRC-TGT devices

pbrun symrcopy create -copy -name 22june2008 -pull -cold -file ssunacs6_map_22june -pace 2 -nop

### ssunacs6_map_22june

#Control Device  Remote Device

###############  #############

symdev=2246:5BC  symdev=1012:0117

symdev=2246:238  symdev=1012:0118

symdev=2246:3BE  symdev=1012:011D

symdev=2246:3C3  symdev=1012:0122

symdev=2246:3C8  symdev=1012:0127

symdev=2246:3CD  symdev=1012:012C

symdev=2246:41A  symdev=1012:0131

symdev=2246:41F  symdev=1012:0136

symdev=2246:424  symdev=1012:013B

symdev=2246:429  symdev=1012:0140

symdev=2246:42E  symdev=1012:0145

symdev=2246:433  symdev=1012:014A

symdev=2246:438  symdev=1012:014F

symdev=2246:43D  symdev=1012:0154

symdev=2246:442  symdev=1012:0159

symdev=2246:45E  symdev=1012:015E

symdev=2246:463  symdev=1012:0163

symdev=2246:28D  symdev=1012:0168

symdev=2246:290  symdev=1012:016B

symdev=2246:4C6  symdev=1012:016E

symdev=2246:2BC  symdev=1153:0181

symdev=2246:468  symdev=1153:0189

symdev=2246:46D  symdev=1153:025C

symdev=2246:472  symdev=1153:0261

symdev=2246:2C9  symdev=1153:02CA

symdev=2246:2F7  symdev=1153:02D2

symdev=2246:368  symdev=1153:02DA

symdev=2246:371  symdev=1153:02E2

symdev=2246:37A  symdev=1153:02EA

symdev=2246:234  symdev=1153:0524

symdev=2246:4C8  symdev=1153:0529

symdev=2246:477  symdev=1153:052B

symdev=2246:47C  symdev=1153:0530

symdev=2246:5BD  symdev=1153:061C

symdev=2246:4CA  symdev=1153:06A6

symdev=2246:4CF  symdev=1153:06AB

symdev=2246:5C3  symdev=1153:072F

symdev=2246:5C9  symdev=1153:0735

symdev=2246:5CF  symdev=1153:0826

14. Start "Copy" session by activating the session

pbrun symrcopy activate -file ssunacs6_map_22june -consistent -nop

15. Set the pace to 2 ( *** We can include this in "symrcopy create" command)

pbrun symrcopy set pace 2 -file  ssunacs6_map_22june -nop

16. Query the session

pbrun symrcopy -file ssunacs6_map_22june query

### Once the copy completes

pbrun symrcopy -file ssunacs6_map_22june verify -copied

17. Terminate the session

pbrun symrcopy -file ssunacs6_map_22june terminate -nop

18. Make the target devices ready.

pbrun symdev -sid 2246 ready -file ssunacs6_2246_tgtdevs -nop

19. Makesure that only devices from "New Frame" are visible to Host.

## Devices from 2246 should be visible

pbrun symmaskdb -sid 2246 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 2246 -wwn 10000000c9436380   list devs

## No devices from 1012 should be visible

pbrun symmaskdb -sid 1012 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 1012 -wwn 10000000c9436380   list devs

## No devices from 1012 should be visible

pbrun symmaskdb -sid 1153 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 1153 -wwn 10000000c9436380   list devs

20. Ask platform to boot the server and do sanity check.

21. Create Sync/Async pair for "New devices" (If it is not done prior)

###2246_pair_2337_2340

Pair is already created last week.

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 que

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 set mode acp_disk -nop

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 est -nop

###Once the status becomes "Synchronized" OR invalid tracks becomes less than 30000,Change the mode into Sync

pbrun symrdf -sid 2246 -rdfg 2 -g ssunacs6_2246 set mode sync -nop

### For Async leg

pbrun symrdf -sid 2246 -rdfg 8 list|grep -v -|grep ^0|awk '{print $1,$2}' >async_current

pbrun symrdf -sid 2246 -f async_current -rdfg 8 disable

pbrun symrdf -sid 2246 -file async_current -rdfg 8 set mode acp_disk

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 set mode acp_disk

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 est

###Once the status becomes "Synchronized", change the mode into Async

pbrun symrdf  -sid 2246 -rdfg 8 list|grep -v -|grep ^0|awk '{print $1,$2}' >async_all

pbrun symrdf  -sid 2246 -file async_all -rdfg 8 set mode async

pbrun symrdf  -sid 2246 -file async_all -rdfg 8 enable

22. COB side configurartion

a. Create mirror session between 2246_2340 new devices

b. Mask BCVs to COB server.

23. After getting confirmation from platform , do cleanup activity

a. Removal of SRC devices from TGT FAs

#1012 devices from #2246 8cA/9cA

pbrun symmask -sid 1012 -wwn 5006048c52a6f987 -dir 7a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 5006048c52a6f988 -dir 10a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 devices to #2246 8cA/9cA

pbrun symmask -sid 1153 -wwn 5006048c52a6f987 -dir 4b -p 0 remove dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

pbrun symmask -sid 1153 -wwn 5006048c52a6f988 -dir 13b -p 0 remove dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

b. Delete Sync/Async pair for 1012_1450_1389 & 1153_1923_1587 devices

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 deletepair -nop

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 deletepair -nop

c. Split HK R2-BCV mirror for 1389 & 1587 devices

d. Remove COB masking for old frames(1389 & 1587 R2-BCV devices)

e. Remove the FA-FA zoning between 1012-2246  and 1153-2246

f. Remove the zoning between Host WWN - 1012&1153 FAs / COB host- 1389 &1587 FAs

OR Migration

Srinadh

## General

Symmetrix Open Replicator

ssunacs6

Host WWNs

10000000c93dece0  #1012-7aA #1153-4bA  #2246-8cA

10000000c9436380  #1012-10aA #1153-13bA  #2246-9cA

1. Identify Target frame/FAs.

2. Get the details of source devices and cut the disks in target frame with same properties(meta config/size) and map to

decided FAs.

3. Get the zoning done for SRC frame FAs(to which the devices are mapped) with TGT frame FAs. (Done)

#1012 FA-7aA 500604844a379106 with 2246 FA-8cA 5006048c52a6f987

#1012 FA-10aA 500604844a379109 with 2246 FA-9cA 5006048c52a6f988

#1153 FA-4bA 5006048452a5e853 with 2246 FA-8cA 5006048c52a6f987

#1153 FA-13bA 5006048452a5e85c with 2246 FA-9cA 5006048c52a6f988

4. Mask the source devices to Target FAs. (Not yet done)

#1012 devices to #2246 8cA/9cA

pbrun symmask -sid 1012 -wwn 5006048c52a6f987 -dir 7a -p 0 add dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 5006048c52a6f988 -dir 10a -p 0 add dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 devices to #2246 8cA/9cA

pbrun symmask -sid 1153 -wwn 5006048c52a6f987 -dir 4b -p 0 add dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

pbrun symmask -sid 1153 -wwn 5006048c52a6f988 -dir 13b -p 0 add dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

5. Mask the TGT devices to Host

pbrun symmask -sid 2246 -wwn 10000000c93dece0  -dir 8c -p 0 add dev 5BC,238,3BE,3C3,3C8,3CD,41A,41F,424,429,42E,433,438,43D,442,45E,463,28D,290,4C6,2BC,468,46D,472,2C9,2F7,368,371,37A,234,4C8,477,47C,5BD,4CA,4CF,5C3,5C9,5CF

pbrun symmask -sid 2246 -wwn 10000000c9436380   -dir 9c -p 0 add dev 5BC,238,3BE,3C3,3C8,3CD,41A,41F,424,429,42E,433,438,43D,442,45E,463,28D,290,4C6,2BC,468,46D,472,2C9,2F7,368,371,37A,234,4C8,477,47C,5BD,4CA,4CF,5C3,5C9,5CF

6. Get the confirmation from Platform team regarding newly added devices.

7. Stop the Application/Bring down the database/Shutdown the server.Get confirmation from platform team.

8. Suspend the SRDF/S for the Source devices to maintain point-in-time copy on the R2 (CT2) site.

9. Create the DGs for SRC/TGT devices to split/establish easily.

pbrun symdg create ssunacs6_1012 -type rdf1

pbrun symld -sid 1012 -g ssunacs6_1012 add dev < 117 & remaining devices also>

## Split the Sync pair of SRC devices

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 que

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 suspend

pbrun symdg create ssunacs6_1153 -type rdf1

pbrun symld -sid 1153 -g ssunacs6_1153 add dev < 181 & remaining devices also>

## Split the Sync pair of SRC devices

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 que

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 suspend

pbrun symdg create ssunacs6_2246 -type rdf1

pbrun symld -sid 2246 -g ssunacs6_2246 add dev < 5BC & remaining devices also>

## Split the Sync pair of TGT devices (If already pair is created for new devices)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 que

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 suspend

## Split the Async pair of TGT devices (If already pair is created for new devices)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 que

(We need to follow Async steps like changing mode of rdfg to acp ..)

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 suspend

10. Un-Mask the source devices from Host.

#1012 Source devices from host

pbrun symmask -sid 1012 -wwn 10000000c93dece0   -dir 7a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 10000000c9436380   -dir 10a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 Source devices from host

pbrun symmask -sid 1153 -wwn 10000000c93dece0 -dir 4b -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1153 -wwn 10000000c9436380 -dir 13b -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

11. Make the Source devices "Write_Disable"  (Pls. be careful when executing this command.Crossverify Frame ID and devices)

pbrun symdev -sid 1012 write_disable -file ssunacs6_1012_srcdevs -nop

##ssunacs6_1012_srcdevs (create all these files under 22june2008)

0117

0118

011D

0122

0127

012C

0131

0136

013B

0140

0145

014A

014F

0154

0159

015E

0163

0168

016B

016E

pbrun symdev -sid 1153 write_disable -file ssunacs6_1153_srcdevs -nop

##ssunacs6_1153_srcdevs (create all these files under 22june2008)

0181

0189

025C

0261

02CA

02D2

02DA

02E2

02EA

0524

0529

052B

0530

061C

06A6

06AB

072F

0735

0826

12. Make the target devices "Not_Ready" (Pls. be careful when executing this command.Crossverify Frame ID and devices)

pbrun symdev -sid 2246 not_ready -file ssunacs6_2246_tgtdevs -nop

5BC

238

3BE

3C3

3C8

3CD

41A

41F

424

429

42E

433

438

43D

442

45E

463

28D

290

4C6

2BC

468

46D

472

2C9

2F7

368

371

37A

234

4C8

477

47C

5BD

4CA

4CF

5C3

5C9

5CF

13. Create Cold-Pull session between SRC-TGT devices

pbrun symrcopy create -copy -name 22june2008 -pull -cold -file ssunacs6_map_22june -pace 2 -nop

### ssunacs6_map_22june

#Control Device  Remote Device

###############  #############

symdev=2246:5BC  symdev=1012:0117

symdev=2246:238  symdev=1012:0118

symdev=2246:3BE  symdev=1012:011D

symdev=2246:3C3  symdev=1012:0122

symdev=2246:3C8  symdev=1012:0127

symdev=2246:3CD  symdev=1012:012C

symdev=2246:41A  symdev=1012:0131

symdev=2246:41F  symdev=1012:0136

symdev=2246:424  symdev=1012:013B

symdev=2246:429  symdev=1012:0140

symdev=2246:42E  symdev=1012:0145

symdev=2246:433  symdev=1012:014A

symdev=2246:438  symdev=1012:014F

symdev=2246:43D  symdev=1012:0154

symdev=2246:442  symdev=1012:0159

symdev=2246:45E  symdev=1012:015E

symdev=2246:463  symdev=1012:0163

symdev=2246:28D  symdev=1012:0168

symdev=2246:290  symdev=1012:016B

symdev=2246:4C6  symdev=1012:016E

symdev=2246:2BC  symdev=1153:0181

symdev=2246:468  symdev=1153:0189

symdev=2246:46D  symdev=1153:025C

symdev=2246:472  symdev=1153:0261

symdev=2246:2C9  symdev=1153:02CA

symdev=2246:2F7  symdev=1153:02D2

symdev=2246:368  symdev=1153:02DA

symdev=2246:371  symdev=1153:02E2

symdev=2246:37A  symdev=1153:02EA

symdev=2246:234  symdev=1153:0524

symdev=2246:4C8  symdev=1153:0529

symdev=2246:477  symdev=1153:052B

symdev=2246:47C  symdev=1153:0530

symdev=2246:5BD  symdev=1153:061C

symdev=2246:4CA  symdev=1153:06A6

symdev=2246:4CF  symdev=1153:06AB

symdev=2246:5C3  symdev=1153:072F

symdev=2246:5C9  symdev=1153:0735

symdev=2246:5CF  symdev=1153:0826

14. Start "Copy" session by activating the session

pbrun symrcopy activate -file ssunacs6_map_22june -consistent -nop

15. Set the pace to 2 ( *** We can include this in "symrcopy create" command)

pbrun symrcopy set pace 2 -file  ssunacs6_map_22june -nop

16. Query the session

pbrun symrcopy -file ssunacs6_map_22june query

### Once the copy completes

pbrun symrcopy -file ssunacs6_map_22june verify -copied

17. Terminate the session

pbrun symrcopy -file ssunacs6_map_22june terminate -nop

18. Make the target devices ready.

pbrun symdev -sid 2246 ready -file ssunacs6_2246_tgtdevs -nop

19. Makesure that only devices from "New Frame" are visible to Host.

## Devices from 2246 should be visible

pbrun symmaskdb -sid 2246 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 2246 -wwn 10000000c9436380   list devs

## No devices from 1012 should be visible

pbrun symmaskdb -sid 1012 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 1012 -wwn 10000000c9436380   list devs

## No devices from 1012 should be visible

pbrun symmaskdb -sid 1153 -wwn 10000000c93dece0   list devs

pbrun symmaskdb -sid 1153 -wwn 10000000c9436380   list devs

20. Ask platform to boot the server and do sanity check.

21. Create Sync/Async pair for "New devices" (If it is not done prior)

###2246_pair_2337_2340

Pair is already created last week.

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 que

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 set mode acp_disk -nop

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 2 est -nop

###Once the status becomes "Synchronized" OR invalid tracks becomes less than 30000,Change the mode into Sync

pbrun symrdf -sid 2246 -rdfg 2 -g ssunacs6_2246 set mode sync -nop

### For Async leg

pbrun symrdf -sid 2246 -rdfg 8 list|grep -v -|grep ^0|awk '{print $1,$2}' >async_current

pbrun symrdf -sid 2246 -f async_current -rdfg 8 disable

pbrun symrdf -sid 2246 -file async_current -rdfg 8 set mode acp_disk

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 set mode acp_disk

pbrun symrdf -sid 2246 -g ssunacs6_2246 -rdfg 8 est

###Once the status becomes "Synchronized", change the mode into Async

pbrun symrdf  -sid 2246 -rdfg 8 list|grep -v -|grep ^0|awk '{print $1,$2}' >async_all

pbrun symrdf  -sid 2246 -file async_all -rdfg 8 set mode async

pbrun symrdf  -sid 2246 -file async_all -rdfg 8 enable

22. COB side configurartion

a. Create mirror session between 2246_2340 new devices

b. Mask BCVs to COB server.

23. After getting confirmation from platform , do cleanup activity

a. Removal of SRC devices from TGT FAs

#1012 devices from #2246 8cA/9cA

pbrun symmask -sid 1012 -wwn 5006048c52a6f987 -dir 7a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

pbrun symmask -sid 1012 -wwn 5006048c52a6f988 -dir 10a -p 0 remove dev 0117,0118,011D,0122,0127,012C,0131,0136,013B,0140,0145,014A,014F,0154,0159,015E,0163,0168,016B,016E

#1153 devices to #2246 8cA/9cA

pbrun symmask -sid 1153 -wwn 5006048c52a6f987 -dir 4b -p 0 remove dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

pbrun symmask -sid 1153 -wwn 5006048c52a6f988 -dir 13b -p 0 remove dev 0181,0189,025C,0261,02CA,02D2,02DA,02E2,02EA,0524,0529,052B,0530,061C,06A6,06AB,072F,0735,0826

b. Delete Sync/Async pair for 1012_1450_1389 & 1153_1923_1587 devices

pbrun symrdf -sid 1012 -g ssunacs6_1012 -rdfg 2 deletepair -nop

pbrun symrdf -sid 1153 -g ssunacs6_1153 -rdfg 2 deletepair -nop

c. Split HK R2-BCV mirror for 1389 & 1587 devices

d. Remove COB masking for old frames(1389 & 1587 R2-BCV devices)

e. Remove the FA-FA zoning between 1012-2246  and 1153-2246

f. Remove the zoning between Host WWN - 1012&1153 FAs / COB host- 1389 &1587 FAs

emcquick.sh

#!/bin/sh

set PATH=$PATH:/usr/sbin

echo Executing ---hostname---;hostname > /tmp/emcquick_hostname.out

echo Executing ---date---; date > /tmp/emcquick_date.out

echo Executing ---uname -a---; uname -a > /tmp/emcquick_uname.out

echo Executing ---set---; set > /tmp/emcquick_set.out

echo Executing ---df -k---; df -k > /tmp/emcquick_df.out

echo Executing ---ifconfig -a---; ifconfig -a > /tmp/emcquick_ifconfig.out

echo Executing ---dmesg---; dmesg> /tmp/emcquick_dmesg.out

echo Executing ---mount---; mount> /tmp/emcquick_mount.out

echo Executing ---ps -aef---;ps -aef> /tmp/emcquick_ps.out

echo Executing ---swap -l---; swap -l> /tmp/emcquick_swap-l.out

echo Executing ---swap -s---; swap -s> /tmp/emcquick_swap-s.out

echo Executing ---pkginfo -l---; pkginfo -l> /tmp/emcquick_pkginfo.out

echo Executing ---prtdiag -v---; prtdiag -v> /tmp/emcquick_prtdiag.out

echo Executing ---prtconf -D---; prtconf -D> /tmp/emcquick_prtconf.out

echo Executing ---prtpicl -v---; prtpicl -v> /tmp/emcquick_prtpicl.out

echo Executing ---kstat---; kstat> /tmp/emcquick_kstat.out

echo Executing ---sysdef -D---; sysdef -D> /tmp/emcquick_sysdef.out

echo Executing ---grep . /kernel/drv/*.conf----

# grep is used, so file name is prepended

grep . /kernel/drv/*.conf> /tmp/emcquick_kernelconf.out

echo Executing ---grep . /etc/system---; grep . /etc/system > /tmp/emcquick_etc_system.out

echo Executing ---date---; date > /tmp/emcquick_date.out

echo DONE Executing. Results are in /tmp/emcquick_\*.out

####

Clariion Connectivity to switches

to Switch A      SPA p0,SPB p1   zone with single/multiple  hba/fcd

to Switch B      SPA p1,SPB p0   zone with single/multiple  hba/fcd

Sym Commands for Regular Usage

/usr/lbin/modprpw -m mintm=0,exptm=0,lftm=0

ttlrep03, ttlcol02

====================

symcfg -sid 136 list -fa ALL  (to list all FA's details)

Adding new LUNs on Primary Node

1. symcfg list

2. symmask list hba

3. powermt display paths

(symdev -sid 137 -sa 7c -p 0 list)

4. symmaskdb -sid 137 -dev 2CB,2D3 list assignments

symdev -sid 137 show 2CB

5. symmask -sid 137 -wwn 210000e08b123886 -dir 7c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210100e08b323886 -dir 8c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210000e08b12341b -dir 9c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210100e08b32341b -dir 10c -p 1 add dev 2CB,2D3

symmask -sid 137 refresh

6. symmaskdb -sid 137 -dev 2CB,2D3 list assignments  (to cross check)

powermt display paths

7. devfsadm -v

powermt display paths

powermt check

8. powermt config

9. powermt display paths

8. syminq |egrep '2CB|2D3'

9. format </dev/rdsk/cxtydzs2>

label

(vxdctl hostid <hostname>  ,  if it is a new host)

10. vxdctl enable

11. vxdisksetup -i cxtydz

vxdg init restore_dg disk01_417=c8t17d16s2

12. vxdg -g omdata_dg adddisk omdatadisk26=c4t16d47s2

vxresize -g aisprod_dg aisapp01 +63g disk09_636 disk10_636

vxassist -g aisprod_dg make aisdata03 50g disk09_636 disk10_636 layout=striped

mkfs -F vxfs -o largefiles /dev/vx/rdsk/aisprod_dg/aisdata03

mkdir -p /ais/data03

Update /etc/vfstab

mount /ais/data03

vgcreate -l 255 -p 255 -s 64 vg_ttliras /dev/dsk/c12t2d2

# lvextend -L 1590080 /dev/vg_restore/backup

Logical volume "/dev/vg_restore/backup" has been successfully extended.

Volume Group configuration for /dev/vg_restore has been saved in /etc/lvmconf/vg_restore.conf

# fsadm -b 1628241920 /backup

fsadm: /etc/default/fs is used for determining the file system type

UX:vxfs fsadm: INFO: V-3-23585: /dev/vg_restore/rbackup is currently 1181351936 sectors - size will be increased

#

Adding new LUNs on Secondry Node

1. symcfg list

2. symmask list hba

3. powermt display paths

4. symmaskdb -sid 137 -dev 2CB,2D3 list assignments

5. symcfg discover

symmask -sid 137 -wwn 210000e08b128003 -dir 7c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210100e08b328003 -dir 8c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210000e08b12c71a -dir 9c -p 1 add dev 2CB,2D3

symmask -sid 137 -wwn 210100e08b32c71a -dir 10c -p 1 add dev 2CB,2D3

symmask -sid 137 refresh

devfsadm -v

powermt config

powermt display paths

vxdctl enable

vxdisk list   (The disks should come online)

Adding BCVs on report server.

1. symcfg list

2. symdg list

symdg show omdbprd_dg

symld -sid 137 -g omdbprd_dg add dev 2CB

symld -sid 137 -g omdbprd dg add dev 2D3

symdg show omdbprd dg

symbcv -sid 137 -g omdbprd_dg associate dev 8A3

symbcv -sid 137 -g omdbprd dg associate dev 8AB

symmir -g omdbprd_dg establish -full DEV023 bcv ld BCV023

symmir -g omdbprd_dg establish -full DEV024 bcv ld BCV024

symdg show omdbprd_dg

symmir -g omdbprd_dg query -i 5

symmir -g omdbprd_dg split

symmir -g omdbprd_dg query -multi

symdg show omdbprd_dg

symmask list hba

symmaskdb -sid 137 -dev 8A3,8AB list assignments

symmask -sid 137 -wwn 210000e08b12ae1b -dir 07B -p 1 add dev 08A3,08AB

symmask -sid 137 -wwn 210000e08b12f01b -dir 08B -p 1 add dev 08A3,08AB

symmask -sid 137 -wwn 210100e08b32f01b -dir 09B -p 1 add dev 08A3,08AB

symmask -sid 137 -wwn 210100e08b32ae1b -dir 10B -p 1 add dev 08A3,08AB

symmask -sid 137 refresh

symmaskdb -sid 137 -dev 8A3,8AB list assignments

devfsadm -v

vxdctl enable

vxdisk list

To find free decice

===================

symmask list hba   (note down the FA ports)

symdev -sid <symid> -sa <director> -p <port> list

on each device

symmaskdb -sid <symid> -dev <dev,dev> list assignments

Mapping devices:   (If unmasked devices are not available)

================

symdev -sid <sid> -noport list  ( to check free decies, i.e not assigned to any FA port)

symdev -sid <sid> show <dev>    (to cross check)

symmask list hba

symcfg -sid <sid> -fa <FA> -p <p> list -address -available   (To check available addresses)

Create map file as below

For SUN:

========

map dev 077B to  dir 7b:1 target=0,lun=00E;

map dev 0783 to  dir 7b:1 target=0,lun=00F;

map dev 078B to  dir 7b:1 target=0,lun=010;

map dev 0793 to  dir 7b:1 target=0,lun=011;

map dev 077B to  dir 10b:1 target=0,lun=00E;

map dev 0783 to  dir 10b:1 target=0,lun=00F;

map dev 078B to  dir 10b:1 target=0,lun=010;

map dev 0793 to  dir 10b:1 target=0,lun=011;

For HP:

=======

map dev 0599 to dir 04c:1 vbus=0,target=3,lun=2;

map dev 05A1 to dir 04c:1 vbus=0,target=3,lun=3;

map dev 05A9 to dir 04c:1 vbus=0,target=3,lun=4;

map dev 05B1 to dir 04c:1 vbus=0,target=3,lun=5;

map dev 05B9 to dir 04c:1 vbus=0,target=3,lun=6;

map dev 05C1 to dir 04c:1 vbus=0,target=3,lun=7;

map dev 0599 to dir 13c:1 vbus=0,target=3,lun=2;

map dev 05A1 to dir 13c:1 vbus=0,target=3,lun=3;

map dev 05A9 to dir 13c:1 vbus=0,target=3,lun=4;

map dev 05B1 to dir 13c:1 vbus=0,target=3,lun=5;

map dev 05B9 to dir 13c:1 vbus=0,target=3,lun=6;

map dev 05C1 to dir 13c:1 vbus=0,target=3,lun=7;

Note :-  In HP we can assign LUNs 0 to 7 only per target.

symcfg discover

symconfigure -sid <sid> verify

symconfigure -sid <sid> -f <map file> preview

symconfigure -sid <sid> -f <map file> prepare

symconfigure -sid <sid> -f <map file> commit

if any step fails

symconfigure -sid 636 verify

symconfigure -sid 636 -f map prepare

symconfigure -sid 636 -f map commit

symdev -sid 0636 list -lock

symcfg -sid 0636 list -lockn

symdev -sid 0636 release -lock

symcfg -sid 0636 release -lockn <lock number>

reboot pc

symconfigure -sid <sid> verify

symdev -sid <symid> -sa <director> -p <port> list  (to cross check)

Note :-  If prepare or commit is taking long time, run verify parallaly

If any step is failed we may need to restart the service processor (laptop).

Unmap devices:

===============

unmap dev 2B3   from dir 7d:1   devmask_access=remove;

unmap dev 2BB   from dir 7d:1   devmask_access=remove;

unmap dev 303   from dir 7d:1   devmask_access=remove;

unmap dev 30B   from dir 7d:1   devmask_access=remove;

unmap dev 28B   from dir ALL:ALL ;

unmap dev 2C3   from dir ALL:ALL;

unmap dev 2CB   from dir  ALL:ALL

Create metas:

=============

form meta from dev 023B config=striped,stripe_size=1920;add dev 23C:242 to meta 23B;

form meta from dev D1B,config=striped,stripe_size=1920;add dev d1c:d22 to meta d1b;

form meta from dev 09D7 config=striped,stripe_size=1920;add dev 09D8:09DA to meta 09D7;

form meta from dev 0A13 config=striped,stripe_size=1920;add dev 0A14:0A1A to meta 0A13;

form meta from dev 0A1B config=striped,stripe_size=1920;add dev 0A1C:0A22 to meta 0A1B;

SYMCLI license:

===============

cat /var/symapi/config/symapi_licenses.dat

License Key: 7FBE-9150-E0BC-9CCD  SYMAPI Feature: BASE

License Key: 3A61-6044-C190-0196  SYMAPI Feature: SERVER

License Key: 4B00-4011-EF38-1609  SYMAPI Feature: TimeFinder

License Key: C7B7-B992-1954-4D99  SYMAPI Feature: DeltaMark

License Key: C0C9-80AF-366B-A846  SYMAPI Feature: SOLUTION_4

License Key: A97E-6602-DABA-9E2A  WIDESKY Feature: DevMasking / Symmetrix

License Key: 8EC5-F23C-F234-F081  SYMAPI Feature: SRDF

Switch Zoning

=============

Create Zone:

0. switchshow

1. zoneshow

2. cfgshow

3. zonecreate "hostname_hba_fa_storage","<WWN of Switch port>;<wwn of Hba>"

4. cfgadd "<switchcfg config>","hostname_hba_fa_storage"

5. cfgsave

6. cfgenable "<switchcfg config>"

Delete Zone:

TTLHYDSW17:admin> zoneremove "OPENREL2","50:06:01:69:30:60:33:32;50:06:04:8c:52:a5:30:57"

TTLHYDSW17:admin> zoneshow OPENREL2

"OPENREL2" does not exist.

TTLHYDSW17:admin> cfgremove "TTLHYDSW17","OPENREL2"

TTLHYDSW17:admin> cfgsave

Updating flash ...

TTLHYDSW17:admin> cfgenable "TTLHYDSW17"

zone config "TTLHYDSW17" is in effect

Updating flash ...

TTLHYDSW21:admin> zonecreate "TTLBPFX1003_hba2_fa10bB_133","50:06:04:84:4a:36:b5:79;21:00:00:e0:8b:12:ab:1b"

TTLHYDSW21:admin> zonecreate "TTLBPFX1003_hba2_fa9bB_130","50:06:04:84:4a:36:b4:b8;21:00:00:e0:8b:12:ab:1b"

TTLHYDSW21:admin> zonecreate "TTLBPFX1003_hba2_spA1_spB0_CX17","50:06:01:68:30:60:37:6a;50:06:01:61:30:60:37:6a;21:00:0

0:e0:8b:12:ab:1b"

TTLHYDSW21:admin> cfgadd "TTLHYDSW21_CONFIG","TTLBPFX1003_hba2_fa10bB_133"

TTLHYDSW21:admin> cfgadd "TTLHYDSW21_CONFIG","TTLBPFX1003_hba2_fa9bB_130"

TTLHYDSW21:admin> cfgadd "TTLHYDSW21_CONFIG","TTLBPFX1003_hba2_spA1_spB0_CX17"

TTLHYDSW21:admin> cfgsave

You are about to save the Defined zoning configuration. This

action will only save the changes on Defined configuration.

Any changes made on the Effective configuration will not

take effect until it is re-enabled.

Do you want to save Defined zoning configuration only?  (yes, y, no, n): [no] yes

Updating flash ...

TTLHYDSW21:admin> cfgenable "TTLHYDSW21_CONFIG"

You are about to enable a new zoning configuration.

This action will replace the old zoning configuration with the

current configuration selected.

Do you want to enable 'TTLHYDSW21_CONFIG' configuration  (yes, y, no, n): [no] yes

zone config "TTLHYDSW21_CONFIG" is in effect

Updating flash ...

TTLHYDSW21:admin> cfgshow

Update qla2300.conf   (example entries)

=====================

hba0-adapter-node-name="200000e08b123d1b";

hba0-adapter-port-name="210000e08b123d1b";

#SPA1 of CX-700-10

hba0-SCSI-target-id-20-fibre-channel-node-name="5006016130603332";

hba0-SCSI-target-id-20-fibre-channel-port-name="5006016130603332";

#SPB1 of CX-700-10

hba0-SCSI-target-id-30-fibre-channel-node-name="5006016930603332";

hba0-SCSI-target-id-30-fibre-channel-port-name="5006016930603332";

#FA9bB of DMX-130

hba0-SCSI-target-id-40-fibre-channel-node-name="500604844a36b4b7";

hba0-SCSI-target-id-40-fibre-channel-port-name="500604844a36b4b7";

#FA10bB of DMX-133

hba0-SCSI-target-id-50-fibre-channel-node-name="500604844a36b579";

hba0-SCSI-target-id-50-fibre-channel-port-name="500604844a36b579";

hba2-adapter-node-name="200000e08b12a21b";

hba2-adapter-port-name="210000e08b12a21b";

#SPA2 of CX-700-10

hba2-SCSI-target-id-21-fibre-channel-node-name="5006016230603332";

hba2-SCSI-target-id-21-fibre-channel-port-name="5006016230603332";

#SPB2 of CX-700-10

hba2-SCSI-target-id-31-fibre-channel-node-name="5006016A30603332";

hba2-SCSI-target-id-31-fibre-channel-port-name="5006016A30603332";

#FA8bB of DMX-130

hba2-SCSI-target-id-41-fibre-channel-node-name="500604844a36b4b8";

hba2-SCSI-target-id-41-fibre-channel-port-name="500604844a36b4b8";

#FA9bB of DMX-133

hba2-SCSI-target-id-51-fibre-channel-node-name="500604844a36b578";

hba2-SCSI-target-id-51-fibre-channel-port-name="500604844a36b578";

Update sd.conf   (example entries)

==============

Note :- You need to add lun 0 to 255 entries for each target.

name="sd" class="scsi" target=20 lun=0;

name="sd" class="scsi" target=20 lun=1;

name="sd" class="scsi" target=20 lun=2;

name="sd" class="scsi" target=20 lun=3;

name="sd" class="scsi" target=20 lun=4;

name="sd" class="scsi" target=20 lun=5;

name="sd" class="scsi" target=20 lun=6;

name="sd" class="scsi" target=20 lun=7;

DR system board  (In this example we are moving SB15 from domain G to H)

=====================

1. Find out the permanent board (You should not delete or touch this board)

To check permanent memory board

(On domain as root)

cfgadm -av |grep -i perm

(On SC as sms-svc, i.e rsh 10.1.1.1)

showplatform -p domains

showdevices -d G   (it should show non-zero)

Note:- If JASS is installed you need to uncommnet below entries in

/etc/inted.conf and run kill -HUP for inetd process.

sun-dr  stream  tcp     wait    root    /usr/lib/dcs    dcs

sun-dr  stream  tcp6    wait    root    /usr/lib/dcs    dcs

2. deleteboard SB15

3. addboard SB15 -d H (target domain)

SUN cluster installation and configuration

==========================================

Quoram device setup

--------------------

scgdevs

scdidadm -L

scsetup

use d14 as quoram device

install CVM

-------------

/usr/cluster/bin/scvxinstall -L <license> -d <dump dir>

encapsulate root disk

install cluster patches

Created dump devices and do root disk mirroring

------------------------------------------------

vxdisksetup -i c0t1d0

vxdg -g rootdg adddisk coredisk=c0t1d0

vxassist -g rootdg make coredump 16g coredisk

vxassist -g rootdg make corevol  16g coredisk

newfs /dev/vx/rdsk/corevol   (default is UFS)

update /etc/vfstab

(/dev/vx/dsk/corevol /dev/vx/rdsk/corevol /core ufs 2 yes logging

mkdir /core

mount /core

vxmksdpart -g rootdg coredisk-01 0 0x0 0x200

vxmksdpart -g rootdg coredisk-02 0 0x0 0x200

dumpadm

dumpadm -d /dev/dsk/c0t1d0s0 -s /core

vxdisksetup -i c2t0d0 old_layout   (to use sector 0 also, i.e new_layout will use from sector 1)

vxdg adddisk rootmir_2=c2t0d0

vxmirror rootdisk_2 rootmir_2

vxmend -g qarac_dg -r clear all applsysx02.dbf applsysx02.dbf-02 -- to clear the locks on plexes

scgdevs    (to register devices)

scconf -a -D type=vxvm,name=eposmig_dg,nodelist=TTLPOSDB01:TTLPOSDB02,preferenced=true

(to register disk group)

scconf -c -D name=eposmig_dg,sync   (to sync veritas dg config)

scswitch -z -D eposmig_dg -h TTLPOSDB01 (to switchover the dg)

---------------------------------------

Unlock user on hp-ux

/usr/lbin/getprpw -l <username>

(in the output if lockout=0001000 means user got locked)

/usr/lbin/modprpw -k <username>  (to unlock user)

(cross check lockout in the output, it should be lockout=0000000)

-------------------------------------------------------------------------

rad command on hp-ux 11.11  (olrad on hp-ux 11iv2)

The rad command provides the ability to perform on-line addition and

replacement of I/O cards.  This command should be used while the

system is in single-user state, when SAM is unavailable.

---------------------------------------------------------------------------------

Clariion License Registration:

root@BPFXUAT # bash

root@BPFXUAT #  powermt set policy=co

Error: "co" is not a valid policy for CLARiiON

storage systems with the installed license.

root@BPFXUAT #  powermt check_registration

There are no license keys now registered.

root@BPFXUAT # emcpreg add BVPA-3B47-YFAW-Q9MQ-MC9F-2M6W

Usage:

emcpreg [opts] -add key [key ...]

emcpreg [opts] -remove key [key ...]

emcpreg [opts] -check key [key ...]

emcpreg [opts] -list

emcpreg [opts] -edit

emcpreg [opts] -install [-noinput]

Options:

-f file     license file

root@BPFXUAT # emcpreg -add BVPA-3B47-YFAW-Q9MQ-MC9F-2M6W

1 key(s) successfully added.

root@BPFXUAT #  powermt set policy=co

root@BPFXUAT #  powermt set policy=co dev=all

root@BPFXUAT # powermt save

-------------------------------------------------------------------------

Note: If EMC luns are not allowing to do vxdisksetup and if it gave error VTOC error. Then first try to scrub with them using scsi command (Cluster software needs to be installed).

Ex: # scdidadm -L |grep < disk number>

from the above note down the did number.

# /usr/cluster/lib/scsi -c inkeys -d <did number followed by s2>

<d44s2>

# If you find any inkeys then execute the below command

# /usr/cluster/lib/scsi -c scrub -d <did number followed by s2>

If you still receive the VTOC error while vxdisksetup.

Then execute

cd /opt/SUNWtscsi/tset_scsi/sa_prog/scsi3

Issue the following command:

do_mhiocgrp_clear /dev/rdsk/<device name>

c4t16d101s2

If we are still finding VTOC Errors while vxdisksetup then

Issue the following command:

./do_mhiocgrp_register -n 2222 /dev/rdsk/<device name>

<c4t16d101s2>

./do_mhiocgrp_preemptandabort -r 2223 -v 1001 /dev/rdsk/<device name>

c4t16d101s2

Finally execute vxdisksetup -i -f <device name>

Veritas 4.1 has got the problem when we add new disks to host from the storage and to over come this problem, please do follow the below command sequence...

symmask -sid <sid> refresh

devfsadm -v

powermt config

powermt save

vxdctl enable

So, we will not get duplicate entries in vxdisk list output.

echo "\033[33m"

vxmake -g fmsrasdev02_dg sd disk02_cx10-07 disk=disk02_cx10 len=57511680 offset=219314944

vxsd -g fmsrasdev02_dg mv fmsrasdev02-03-02 disk02_cx10-07 &

rsize=32768,wsize=32768,hard

scswitch -F -g fxka-rg

scswitch -F -D fxkaraw_dg

scswitch -z -D fxkaraw_dg -h TTLBPFX05

scswitch -z -g fxka-rg -h TTLBPFX05

*************************************************

vgreduce -l /dev/vg00  /dev/dsk/c0t6d0

prtfru "/frutree/chassis/CP/ex8?Label=ex8/EXB/sb8?Label=sb8/CPU" | grep -i speed

ipcs -s

bash-3.00# ipcs -s | grep 0x45

s      22 0x45187c9c --ra-ra-ra-      root       sys

s      25 0x450c08ea --ra-ra-ra-      root       sys

s   70683 0x451884b3 --ra-ra-ra-      root       sys

s   20508 0x450c0c69 --ra-ra-ra-      root       sys

s   20509 0x450c0d45 --ra-ra-ra-      root       sys

s   76831 0x45187bcd --ra-ra-ra-      root       sys

ipcrm -s 22

new commands..

#symmaskdb -sid <> list database

#symmaskdb -sid <> backup -file <filename>

to replace old wwn with new wwn

#symmask -sid <> -wwn <old wwn> replace  <new wwn>

#symmaskdb -sid <> -wwn <> list devs

#symmaskdb -sid <> list capacity -host <HOSTNAME>  ....# ref symmask list logins

#symmaskdb -sid -devs <>,<>,<> list assignments

to write disable the FA ports

#symld -g DgName -SA 16A -p 0 write_disable

To make symdg "not ready " mode

#symld -sid <> -g <> not_ready

To make SYMdev ""not ready " mode

#symdev -sid <> not_ready <symdev>

to create the hyper:

create dev, count=4, size=220, emulation=fba, config=2-way-mir;

To add four new Symmetrix devices to Symmetrix array 000000012345 as two-way

mirrored devices with a size of 1100 cylinders (516 MB), using FBA emulation, and to

create the hypers to support the new devices in disk group 3, create a command file

containing the following option:

create dev count=4, size=1100,emulation=FBA, config=2-Way-Mir disk_group=3;

To commit the command file that creates a concatenated metadevice in Symmetrix

12345, using device 0010 as the metahead, and 0011 - 0013 as members, enter:

form meta from dev 0010, config = concatenated;

add dev 0011:0013 to meta 0010;

To form a concatenated metadevice 0010 and add metamembers, enter:

form meta from dev 0010, config = concatenated;

add dev 0011:0012 to meta 0010;

Example: Forming a striped meta

To form a striped metadevice 0010 and add metamembers, enter:

form meta from dev 0010, config = striped,stripe_size=1920;

add dev 0013:0014 to meta 0010;

to see the freespace format wise,

#symconfigure -sid <> list -freespace -units MB

When unmapping a device, there can be no I/O activity in the specified mapped

path. If you want to unmap only one path to a device that has paths to multiple

front-end directors, you can write-disable just that path. The symdev command

write-disables the device-0001 path specified by director 04B, port 0.

#symdev –sid 120 write_disable 0001 –sa 04B –p 0

Write Disable operation successfully completed for the device.

Example:

013 Not Visible ???:? 02B:C3 2-Way Mir N/Grp'd (M) RW 960

014 Not Visible ???:? 01A:C0 2-Way Mir N/Grp'd (m) RW -

In this example, the ???:? displayed in the SA :P column for device 013 indicates

that the metadevice is not yet mapped to a port. Subsequent commands will map

the metadevice to a new host connected to front-end director 03A, port 0. The

symcfg list command displays the current port settings (flags) for this port.

#symcfg -sa 03A –p 0 list –v –sid 40

Dynamic RDF:-

===============

vi set_sym.cmd

set symmetrix dynamic_rdf=enable;

#symconfigure –sid 605 –file set_sym.cmd –v –noprompt commit

vi create_std.cmd

create dev, count=6, size=1100, emulation=fba, config=RAID-5;

#symconfigure -sid 605 -file create_std.cmd -v -noprompt commit

The following command uses the vi text editor to create a text file named

set_dyn_devices.cmd. As was done here, you can enter into the file the set dev

entry that sets the dynamic RDF attribute for the six new devices. The dyn_rdf

attribute allows a device to be dynamically converted to either an R1 or an R2

device.

vi set_dyn_devices.cmd

set dev 0091:0096 attribute=dyn_rdf;

#symconfigure -sid 605 -file set_dyn_devices.cmd -v -noprompt commit

vi create_pair.cmd

0091 0010

0092 0011

0093 0012

0094 0013

0095 0014

0096 0015

symdg create new_dyn_rdf_grp –type regular

symld –g new_dyn_rdf_grp –sid 605 addall dev –range 091:096

symld –g new_dyn_rdf_grp write_disable

symrdf -g new_dyn_rdf_grp establish -noprompt -sid 605

symrdf -g new_dyn_rdf_grp query -sid 605

symrdf verify -g new_dyn_rdf_grp -i 30 -sid 605

symrdf createpair -file create_pair.cmd -sid 605 -rdfg 1 -type RDF1 -invalidate R2 -noprompt

==================================

symrdf -g posedm_dg -bcv query

symrdf -g posedm_dg -bcv set mode acp_disk

symrdf -g posedm_dg -bcv establish -nop

Configuring Gatekeeper Devices

You need a gatekeeper device for each device group in the consistency

group that will be built in a later step. Use the following commands on

all nodes on the R1 side to define gatekeepers and associate them with

device groups:

# symgate -sid 638 define dev 010

# symgate -sid 130 define dev 009

# symgate -sid 638 -g dgoraA associate dev 010

# symgate -sid 130 -g dgoraB associate dev 009

Use the following commands on all nodes on the R2 side to define

gatekeepers and associate them with device groups:

# symgate -sid 021 define dev 002

# symgate -sid 363 define dev 00B

# symgate -sid 021 -g dgoraA associate dev 002

# symgate -sid 363 -g dgoraB associate dev 00B

Technical Tipps:

xargs echo | tr  ": :" ":,:"

cat test | xargs echo | tr ": :" ":,:"

bdf | grep -v Filesystem|tr " " " "  cut -d " " -f 3 | sort -rn | xargs echo | sed "s/ /+/g"|bc

mminfo  -avot -t "07/05/07" |grep -i cb | awk '{sum+=$5} END {print sum/1024}'

Meta size Symmetrix

cylinder size = 480 k

Meta Configuration       : Striped

Meta Stripe Size         : 960k    (2 Cylinders)

Mirror Set Type          : [RAID-5,RAID-5,N/A,N/A]

Mirror Set DA Status     : [RW,RW,N/A,N/A]

Solution Enable commands:

To examin the Symmetrix Engunity version

# symcfg list -v

Symcli Avoidence and Selection file: (These files are used to limit the scope of symcli online commands)

/var/symapi/config/symavoid: -> Affects the "symcfg disc" operation so that it doesn't look for devices under the specified box

(Specify the 12 digit sym id in one line)

/var/symapi/config/inqfile -> The inqfile file configures calls to "symcfg discover" and "syminq" to find only those

PdevNames specified in this file. This may be useful if you want to limit the command(s) to

view only certain Symmetrix devices from your host. (Specify one PDevName (/dev/rdsk/c2t0d2s2)

per line.

/var/symapi/config/gkavoid -> This file is used by several symcli command to avoid using the gate keepers specified in this file. (Specify one PDevName

(/dev/rdsk/c2t0d2s2) per line)

/var/symapi/config/gkselect -> This file is used by several symcli command to force using only the gate keepers specified in this file. (Specify one PDevName

(/dev/rdsk/c2t0d2s2) per line)

Monitor Symmetrix events thru SYMCLI:

# symevent monitor

eg) symevent monitor -sid 901 -i 600 -c 144 -warn => i=seconds; c=count; -warn = severity

# symevent list

eg) symevent list -sid 901 -v -start 09:00 -end 17:00

## SRDF

SRDF Sync/Async Activities

(SYNC LEG)

1. pbrun symrdf -sid 2246 createpair -f 04thMay2008r1tor2 -rdfg 2 -type rdf1 -invalidate r2

2. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 query

3. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 set mode acp_disk -nop

4. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 query

5. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 est

once invalid tracks becomes less than 30000 change the mode to Sync. Better wait till invalid tracks become NULL.

6. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 set mode sync

7. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 query

(ASYNC LEG)  -

1. pbrun symrdf -sid 2246 -f 2246_2340_hprdfa_8 -rdfg 8 disable -nop

2. pbrun symrdf -sid 2246 -f 2246_2340_hprdfa_8 -rdfg 8 set mode a acp_disk -nop

3. pbrun symrdf -sid 2246 createpair -f sunrdfa_04thMay2008 -rdfg 8 -type rdf1 -invalidate r2

4. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 8 query

5. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 8 set mode  acp_disk -nop

6. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 8 query

7. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 8 est

8. run the update.sh script:   ( U can this script at end once all frames done)

once invalid tracks becomes less than 30000 change the mode to Sync. Better wait till invalid tracks become NULL.

9. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 2 set mode async

10.  pbrun symrdf -sid 2246 -f 2246_2340_hprdfa_8 -rdfg 8 enable -nop

11. pbrun symrdf -sid 2246 -f 04thMay2008r1tor2 -rdfg 8 query

List all RDF groups in a box

# symcfg -sid <sid> -rdfg all list

List all the RA  ports in a box

# symcfg -ra all list

List all dynamic devices visible to that host

# symdev list pd -dynamic

Creating an RDF group

symrdf addgrp -label <rdf group name> -rdfg <RDF group # (1-64)> -sid <sid> -dir <dir1,dir2> -remote_sid <sid> -remote_dir <remote dir1, remote dir2> -remote_rdfg <RDF group#(keep same as above>

Creating R1 and R2 paring relation

Create a file containing the R1 devices in the first column and R2 devices in 2nd column

101    401

201    501

#symrdf -file <the above file name> -sid <R1 side sid> -rdfg <RDF group #> -type r1 -establish -g <Device Group name> createpair -nop

Deleting the R1 and R2 paring

Create a file containing the R1 devices in the first column and R2 devices in 2nd column

101    401

201    501

#symrdf deletepair -file <the above file name> -sid <R1 sid> -rdfg <RDF roup #>

List all the devices in a RDF group

#symrdf -rdfg <RDF group # | all> list

Asynchronous SRDF mode:

Statistics of an SRDF/A session

symstat -type cache -reptype rdfa -sid <sym id>  -i 5 -c 10

symrdf Query for Async mode

symrdf -g <dg> query -rdfa

List the remote storage association:

symcfg -sid 2538 -ra all list

symrdf -sid 133 ping

Preparing to Convert device for SRDF:

Put the R2 device in unmapped, unmasked, Write Disabled / NR

symdev -sid 2538 show 01E3

symcfg disc

symmaskdb -sid 2538 -dev 01E3 list assignment

symdev -sid 2538 not_ready 01E3 (May not be required)

map file:

convert dev 0293<STD> to RDF1-Mir(For 2-Way Mir), ra_group=3, remote_dev=01E3<R2> invalidate=r2, start_copy=no;

convert dev 09CB to RDF1-Bcv-Mir(For 2-Way BCV Mir), ra_group=3, remote_dev=08EB invalidate=r2, start_copy=no;

convert dev 11B3 to RDF1-BCV(For BCV), ra_group=3, remote_dev=091B invalidate=r2, start_copy=no;

convert dev 11b1 to RDF1+R-5(For RAID-5),ra_group=4, remote_dev=1253, invalidate=R2, start_copy=no;

Create sym DG:

symdg create -type rdf1 fxbhuparbor_dg

symld -g fxbhuparbor_dg  -sid 133 add dev 0293

symrdf -g fxbhuparbor estab

If you are getting error like "The Symmetrix configuration could not be loaded for a remotely-attached Symmetrix"

symrdf -g fxbhuparbor split

symrdf -g fxbhuparbor estab

Set Adaptive Copy and Disk Mode:

symrdf -g fxpbarbor_dg set mode acp_disk(Adaptive copy Disk mode)

symcfg -sid 417 -lockn 15 release -force -nop

symclone -sid 417 -f epos-clone.txt create -precopy -diff

## SYMCLI

symcfg list -connections  -sorthost -capacity

the space mapped to fa port and which host is using that space

Host               Symmetrix                     Capacity (GB)

------------  --------------------------  -----------------------------------

Node Name     ID           Director Port  Mapped STDs Mapped BCVs Paired BCVs

------------  ------------ -------- ----  ----------- ----------- -----------

AISPROD01     000287890636 FA-7B     1         1770.1         0.0         0.0

000287890636 FA-10B    1         1770.1         0.0         0.0

----------- ----------- -----------

AISPROD01 totals:                              1770.1         0.0         0.0

AMDTEST       000287890636 FA-9A     1          748.9      5718.6         0.0

000287890636 FA-13A    0         1565.9      1497.7         0.0

----------- ----------- -----------

AMDTEST totals:                                2110.5      6739.8         0.0

symcfg list -connections

will show the information about the hosts connected to FAs

Symmetrix ID : 000287890130

Symmetrix                                Host

-------------  -----------------------------------------------------------

Director Port  Node Name     IP Address      HW Type  OS Name  OS Revision

-------- ----  ------------- --------------- -------- -------- -----------

DF-1A     N/A  CK287890130   172.17.4.192    INTEL    WinNT-SP 5.0.2195

FA-8A     1    INTECREP      172.17.7.164    9000/800 HPUX     B.11.11

FA-7B     0    ICHARGEDB01   172.17.7.238    sun4u    SunOS    5.10

0    ichargedb01   172.17.7.238    sun4u    SunOS    5.10

0    ichargedb02   172.17.7.248    sun4u    SunOS    5.10

FA-8B     1    TTLHYDSN02    172.17.7.144    sun4u    SunOS    5.9

1    TTLHYDSN03    172.17.7.145    sun4u    SunOS    5.9

symcfg list -lockn all

S Y M M E T R I X      L O C K S

Lock       Lock    Lock                Time

SymmID       Attachment   Status     Number  Usage               Held (Sec)

000287890130 Local        N/A        N/A     N/A                 N/A

symcfg list -fa all -port

gives information about fa port is connected to san switch or not.Director status/port status/connection status

Symmetrix ID: 000287890130 (Local)

S Y M M E T R I X    D I R E C T O R    P O R T S

Director   Port Status     Connection Status

Ident  Type          Status    P0  P1  P2  P3    P0  P1  P2  P3

FA-7A  FibreChannel  Online    ON  ON  N/A N/A    X   X   -   -

FA-8A  FibreChannel  Online    ON  ON  N/A N/A    X   X   -   -

symcfg list -fa all

gives information of all fa ports wwn/vbit status/vcm enabled

Symmetrix ID: 000287890130 (Local)

S Y M M E T R I X    F I B R E   D I R E C T O R S

Dir    Port  WWN               VCM      Volume Set   Pnt to Pnt

Enabled  Addressing

FA-7A   0    500604844a36b486  Yes      No           Yes

FA-7A   1    500604844a36b4a6  Yes      Yes          Yes

symcfg list -ra all -port

gives information about ra groups

Symmetrix ID: 000290100980 (Local)

S Y M M E T R I X    D I R E C T O R    P O R T S

Director   Port Status     Connection Status

Ident  Type          Status    P0  P1  P2  P3    P0  P1  P2  P3

RF-8D  RDF-R1        Online    ON  N/A N/A N/A    X   -   -   -

-   000290102632  2 (01)  2 (01)

RF-9D  RDF-R1        Online    ON  N/A N/A N/A    X   -   -   -

-   000290102632  2 (01)  2 (01)

Legend for Connection Status:

(X) : Fibre Port is Connected to a Fibre Port (HBA, Switch or RF Director)

(-) : Fibre Port is Not Connected.

symcfg list -memory

Symmetrix ID            : 000287890130

Number of Memory Boards : 6

Slot Number            Capacity(MB)

0                       16384

1                       16384

2                       16384

3                        8192

4                        8192

5                        8192

--------

Total (Usable Cache)      73728

--------

symcfg list -rdfg all

Symmetrix ID : 000290100980

S Y M M E T R I X   R D F   G R O U P S

Local             Remote                   Group               RDFA Info

-------------  --------------------  -----------------------  -----------------

LL                                      Flags  Dir  Flags  Cycle

RA-Grp  (sec)  RA-Grp  SymmID        T    Name     LPDS  Cfg   CSRM   time  Pri

-------------  --------------------  -----------------------  -----  -----  ---

2 ( 1)    10   2 ( 1) 000290102632  S BILLGRP4    .X..  F-S   -IS-     30   33

Legend:

?                 : Unknown

Group (T)ype      :  S = Static, D = Dynamic

Director (C)onfig :  F-S = Fibre-Switched, F-H = Fibre-Hub

G = GIGE, E = ESCON, T = T3, - = N/A

Group Flags       :

Prevent Auto (L)ink Recovery      : X = Enabled, . = Disabled

Prevent RAs Online Upon (P)ower On: X = Enabled, . = Disabled

Link (D)omino                     : X = Enabled, . = Disabled

(S)TAR mode                       : N = Normal, R = Recovery, . = OFF

RDFA Flags        :

(C)onsistency : X = Enabled, . = Disabled, - = N/A

(S)tatus      : A = Active, I = Inactive, - = N/A

(R)DFA Mode   : S = Single-session, M = MSC, - = N/A

(M)sc Cleanup : C = MSC Cleanup required, - = N/A

symcfg -sid 417 list -dir all -v

Gives information about all the directors available in box

Symm Commands:

* List all the box visible to this host

symcfg list

* List all the HBAs of this host connected to SAN

symmask list hba

* Lists all the hostname using this box

symcfg -sid 137  list -connections

* List all the box visible to the host and all the hostname using the box

symcfg list -connections

* Shows the Std & BCV LUNS on this DG

symdg show sapdbedm_dg

* Show information of one particular LUN

symdev -sid 636 show 0C93

* Lists all the luns mapped to hosts with hostname and the FA

symmaskdb -sid 1168 list database

* List the WWN of a port

symcfg -sid 636 -fa 13a -p 1 list

* Lists all the FAs with their WWN

symcfg -sid 980 -fa all list

* List all LUNS that are visible to an FA

symdev -sid 636 -sa 13a -p 1 list

* List all bcv that are not assigned to any FA

symdev -sid 636 -meta -bcv -noport list

* List all the assignments of a particular LUN

symmaskdb -sid 636 -dev B03 list assignments

* Display the WWNs of hosts that are using this port( Logged On IN & Fabric => "Yes" => currently using)

symmask -sid 417 -dir 9c -p 1 list logins

* Display dev from the host and physical path

# syminq -pdevfile

* List HBA information

syminq hba -fiber

Change Device configuration:

convert dev 0AB3 to 2-Way-Bcv-Mir;

convert dev 17F1 to BCV+R-5;

SYMMIR Operation:

Split One pair of LUNS <Std - BCV>

symmir -g bptnedm_dg split DEV005 bcv ld BCV005 -force -symforce

Full sync for one pair of LUN

symmir -g bptnedm_dg establish DEV005 -full

Forced sync

symmir -g bptnedm_dg establish DEV005 -full -force

==================== If disks are in Invalid state   ===================

root@TTLBILL02 # symmir -g bptnedm_dg query

Device Group (DG) Name: bptnedm_dg

DG's Type             : REGULAR

DG's Symmetrix ID     : 000287890130

Standard Device                    BCV Device                  State

-------------------------- ------------------------------------- ------------

Inv.                                  Inv.

Logical        Sym  Tracks Logical              Sym       Tracks STD <=> BCV

-------------------------- ------------------------------------- ------------

DEV001         0123      0 BCV001               0BB7 *         0 Split

DEV002         012B      0 BCV002               0BBF *         0 Split

DEV003         0193      0 BCV003               0C17 *         0 Split

DEV004         0093      0 BCV004               0B47 *         0 Split

DEV005         020B      0 BCV005               0C7F *   1365958 Invalid

DEV006         022B      0 BCV006               0C9F *         0 Split

DEV007         0333      0 BCV007               0D87 *         0 Split

DEV008         0113      0 BCV008               0BA7 *         0 Split

DEV009         011B      0 BCV009               0BAF *         0 Split

DEV010         0213      0 BCV010               0C87 *         0 Split

DEV011         021B      0 BCV011               0C8F *         0 Split

DEV012         0403      0 BCV012               0DD7 *         0 Split

DEV013         0203      0 BCV013               0C77 *         0 Split

DEV014         033B      0 BCV014               0D8F *         0 Split

DEV015         040B      0 BCV015               0DDF *         0 Split

DEV016         009B      0 BCV016               0B4F *         0 Split

DEV017         00BF      0 BCV017               0B63 *         0 Split

DEV018         00FF      0 BCV018               0BD3 *         0 Split

DEV019         0153      0 BCV019               0E6F *         0 Split

Total              -------                               -------

Track(s)               0                               1365958

MB(s)                0.0                               42686.2

root@TTLBILL02 # symmir -g bptnedm_dg split DEV005 bcv ld BCV005

Execute 'Split' operation for device 'DEV005'

in device group 'bptnedm_dg' (y/[n]) ? y

'Split' operation execution is in progress for device 'DEV005'

paired with BCV device 'BCV005' in

device group 'bptnedm_dg'. Please wait...

Cannot proceed when the BCV state is 'Sync In Progress' except if the Symmetrix force flag is used

root@TTLBILL02 # symmir -g bptnedm_dg split DEV005 bcv ld BCV005 -symforce

Execute 'Split' operation for device 'DEV005'

in device group 'bptnedm_dg' (y/[n]) ? y

'Split' operation execution is in progress for device 'DEV005'

paired with BCV device 'BCV005' in

device group 'bptnedm_dg'. Please wait...

'Split' operation successfully executed for device 'DEV005'

in group 'bptnedm_dg' paired with BCV device 'BCV005'.

root@TTLBILL02 #

root@TTLBILL02 #

root@TTLBILL02 #

root@TTLBILL02 # symmir -g bptnedm_dg query

Device Group (DG) Name: bptnedm_dg

DG's Type             : REGULAR

DG's Symmetrix ID     : 000287890130

Standard Device                    BCV Device                  State

-------------------------- ------------------------------------- ------------

Inv.                                  Inv.

Logical        Sym  Tracks Logical              Sym       Tracks STD <=> BCV

-------------------------- ------------------------------------- ------------

DEV001         0123      0 BCV001               0BB7 *         0 Split

DEV002         012B      0 BCV002               0BBF *         0 Split

DEV003         0193      0 BCV003               0C17 *         0 Split

DEV004         0093      0 BCV004               0B47 *         0 Split

DEV005         020B      0 BCV005               0C7F *         0 SplitBfrSync

DEV006         022B      0 BCV006               0C9F *         0 Split

DEV007         0333      0 BCV007               0D87 *         0 Split

DEV008         0113      0 BCV008               0BA7 *         0 Split

DEV009         011B      0 BCV009               0BAF *         0 Split

DEV010         0213      0 BCV010               0C87 *         0 Split

DEV011         021B      0 BCV011               0C8F *         0 Split

DEV012         0403      0 BCV012               0DD7 *         0 Split

DEV013         0203      0 BCV013               0C77 *         0 Split

DEV014         033B      0 BCV014               0D8F *         0 Split

DEV015         040B      0 BCV015               0DDF *         0 Split

DEV016         009B      0 BCV016               0B4F *         0 Split

DEV017         00BF      0 BCV017               0B63 *         0 Split

DEV018         00FF      0 BCV018               0BD3 *         0 Split

DEV019         0153      0 BCV019               0E6F *         0 Split

Total              -------                               -------

Track(s)               0                                     0

MB(s)                0.0                                   0.0

Legend:

(*): The paired BCV device is associated with this group.

root@TTLBILL02 # symmir -g bptnedm_dg establish DEV005 -full

Execute 'Full Establish' operation for device 'DEV005'

in device group 'bptnedm_dg' (y/[n]) ? y

'Full Establish' operation execution is in progress for device 'DEV005' in

device group 'bptnedm_dg'. Please wait...

Cannot proceed when the BCV state is 'Split Before Sync' except if the force flag or exact pairing is used

root@TTLBILL02 # symmir -g bptnedm_dg establish DEV005 -full -force

Execute 'Full Establish' operation for device 'DEV005'

in device group 'bptnedm_dg' (y/[n]) ? y

'Full Establish' operation execution is in progress for device 'DEV005' in

device group 'bptnedm_dg'. Please wait...

Partial Failure; BCV Lib error is: pair #8, The specified BCV device is not ready or is write disabled

root@TTLBILL02 # symmir -g bptnedm_dg query -i 5

Device Group (DG) Name: bptnedm_dg

DG's Type             : REGULAR

DG's Symmetrix ID     : 000287890130

Standard Device                    BCV Device                  State

-------------------------- ------------------------------------- ------------

Inv.                                  Inv.

Logical        Sym  Tracks Logical              Sym       Tracks STD <=> BCV

-------------------------- ------------------------------------- ------------

DEV001         0123      0 BCV001               0BB7 *         0 Split

DEV002         012B      0 BCV002               0BBF *         0 Split

DEV003         0193      0 BCV003               0C17 *         0 Split

DEV004         0093      0 BCV004               0B47 *         0 Split

DEV005         020B      0 BCV005               0C7F *   1942131 Invalid

DEV006         022B      0 BCV006               0C9F *         0 Split

DEV007         0333      0 BCV007               0D87 *         0 Split

DEV008         0113      0 BCV008               0BA7 *         0 Split

DEV009         011B      0 BCV009               0BAF *         0 Split

DEV010         0213      0 BCV010               0C87 *         0 Split

DEV011         021B      0 BCV011               0C8F *         0 Split

DEV012         0403      0 BCV012               0DD7 *         0 Split

DEV013         0203      0 BCV013               0C77 *         0 Split

DEV014         033B      0 BCV014               0D8F *         0 Split

DEV015         040B      0 BCV015               0DDF *         0 Split

DEV016         009B      0 BCV016               0B4F *         0 Split

DEV017         00BF      0 BCV017               0B63 *         0 Split

DEV018         00FF      0 BCV018               0BD3 *         0 Split

DEV019         0153      0 BCV019               0E6F *         0 Split

Total              -------                               -------

Track(s)               0                               1942131

MB(s)                0.0                               60691.6

Synchronization rate           :      4.7 MB/S

Estimated time to completion   : 03:36:39

Legend:

(*): The paired BCV device is associated with this group.

=======================  Wait till the sync completes

root@TTLBILL02 # symmir -g bptnedm_dg split DEV005 -force

Execute 'Split' operation for device 'DEV005'

in device group 'bptnedm_dg' (y/[n]) ? y

'Split' operation execution is in progress for device 'DEV005' in

device group 'bptnedm_dg'. Please wait...

'Split' operation successfully executed for device 'DEV005'

in group 'bptnedm_dg'.

SYMCLONE Operation:

List all the symclone sessions for specified box

# symclone -sid 980 list

Symclone for Backups:

symclone -sid 980 -file bpfxtn_data create -copy -diff

Dbase Begin backup mode

symclone -sid 980 -file bpfxtn_data activte

Database End backup mode

symclone -sid 980 -file bpfxtn_arch -create -copy -diff

symclone -sid 980 -file bpfxtn_arch activate

vxdctl enable

vxdg -fC import fxtnraw_dg

vxdg -fC import fxtnarch_dg

vxdg list

Storapid Options:

Enable Debugging for Sym commands:

Enabling high debug level for SYMAPI for Nwr PowerSnap backups.

# nsrps_shutdown -> Shutdown the ps daemon

# export SYMCLI_DEBUG=yes

--> All the below option are possible

# nsrpsd -> Start the ps daemon

******* For High Debug ********

Edit file /var/symapi/config/options and add the following entries:

SYMAPI_DEBUG=-1

SYMAPI_DEBUG1_CATEGORIES=-1

SYMAPI_DEBUG2_CATEGORIES=-1

SYMAPI_DEBUG_FILENAME=<path of some log filename>

Note: Symapi debug logs are really huge. Create the log file on a non-root filesystem with adequate space.

Disable storapid daemon from starting:

Stop te storapid daemon

Edit file /var/symapi/config/daemon_options and add the below line

storapid:client_use = never

Creating LUNs

form meta from dev 453, config=striped, stripe_size=3840;

add dev 454:45a to meta 453;

form meta from dev 45b, config=striped, stripe_size=3840;

add dev 45c:462 to meta 45b;

form meta from dev 463, config=striped, stripe_size=3840;

add dev 464:46a to meta 463;

form meta from dev 46b, config=striped, stripe_size=3840;

add dev 46c:472 to meta 46b;

Mapping and Masking LUNs to a host

1) Check if the devices are masked to other FA

#symmaskdb -sid 133 -dev 0C93,0C9B,0CA3,0CAB,0CB3,0CBB,0CC3,0CCB,0CD3,0CDB,0CE3,0CEB,0CF3,0CFB,0D03,0D0B,0D13,0D1B,0D23,0D2B,0D33,0D3B,0D43,0D4B,0D53,0D5B,0D63,0D6B,0D73,0D7B,0D83,0D8B,0D93,0D9B,0DA3,0DAB,0DB3,0DBB,0DC3,0DCB,0DD3 list assignments

2) Find the available Hexa address to be assigned to the LUNs

# symcfg -sid 133 -fa 7b -p 1 list -address -available

3) Prepare map file

map dev 0C93 to dir 7b:1 target=0, lun=10 (Hexa Value);

map dev 0C9B to dir 7b:1 target=0, lun=11;

map dev 0CA3 to dir 7b:1 target=0, lun=12;

For HP:

map dev 0ADB to dir 9a:1 vbus=0<0-f>, target=9<0-f>, lun=4<0-7>;

4) Apply the config

#symconfigure -sid <sid> verify

#symconfigure -sid <sid> -f <map file> preview

#symconfigure -sid <sid> -f <map file> prepare

#symconfigure -sid <sid> -f <map file> commit

5) Mask the LUNs to the host's HBA

# symcfg disc

#symmask list hba

#symmask -sid 133 -wwn 210000e08b94dd8c -dir 7b -p 1 add dev 0C93,0C9B,0CA3,0CAB,0CB3,0CBB,0CC3,0CCB,0CD3,0CDB,0CE3,0CEB,0CF3,0CFB,0D03,0D0B,0D13,0D1B,0D23,0D2B,0D33,0D3B,0D43,0D4B,0D53,0D5B,0D63,0D6B,0D73,,,0D7B,0D83,0D8B,0D93,0D9B,0DA3,0DAB,0DB3,0DBB,0DC3,0DCB,0DD3

#symmask -sid 133 refresh

#devfsadm -v

#symcfg disc

6) Verify the LUNs are masked

#symmaskdb -sid 133 -dev 0C93,0C9B,0CA3,0CAB,0CB3,0CBB,0CC3,0CCB,0CD3,0CDB,0CE3,0CEB,0CF3,0CFB,0D03,0D0B,0D13,0D1B,0D23,0D2B,0D33,0D3B,0D43,0D4B,0D53,0D5B,0D63,0D6B,0D73,0D7B,0D83,0D8B,0D93,0D9B,0DA3,0DAB,0DB3,0DBB,0DC3,0DCB,0DD3 list assignments

#symdev -sid 133 list pd

Mapping and Masking:

map dev 13BB to dir 07C:0 target=0,lun=5,wwn=210000e08b9425a9;

symmask -sid 136 refresh

Unmapping Devs:

unmap dev <dev id> from dir ALL:ALL;

Rename the WWN number of HBA with HOSTNAME in logins history of Symmetrix Storage.

# symmask disc hba -rename

SYM commands with outputs

Useful commands in alphabetical order

symaudit -sid 417 show

root@INSPROD01 # symaudit -sid 417 show

A U D I T   L O G   D A T A

Symmetrix ID            : 000290100417

Starting date             : 06/08/2007 19:42:14

Ending date              : 02/04/2008 02:11:14

Starting record number  :   73217

Ending record number    :  135087

Total record count      :   61871

root@INSPROD01 # symaudit list -sid 417 -v -start_date 01/22/08 -end_date 01/23/08

A U D I T   L O G   D A T A

Symmetrix ID            : 000290100417

Record Number         :   133816

Records in Seq      :        6

Offset in Seq       :        1

Time                : 01/22/08 03:41:44

Vendor ID           : EMC Corp

Application ID      : SYMCLI

Application Version : 6.0.1.0

API Library         : SEK

API Version         : V6.0.1.0 (Edit Level: 629)

Host Name           : TTLREP03

OS Name             : SunOS

OS Revision         : 5.9Generic

Client Host         :

Process ID          : 00029628

Task ID             : 00000001

Function Class      : CLONE

Action Code         : Recreate

Text                : STARTING a local 'Snap Recreate' operation for 80 [STD-BCV] Pairs:

Record Number         :   133817

Records in Seq      :        6

Offset in Seq       :        2

Time                : 01/22/08 03:41:44

Vendor ID           : EMC Corp

Application ID      : SYMCLI

Application Version : 6.0.1.0

API Library         : SEK

API Version         : V6.0.1.0 (Edit Level: 629)

Host Name           : TTLREP03

OS Name             : SunOS

OS Revision         : 5.9Generic

Client Host         :

Process ID          : 00029628

Task ID             : 00000001

Function Class      : CLONE

Action Code         : Recreate

Text                : SymmID 000290100417:[ 0453-1781 0454-1782 0455-1783 0456-1784 0457-1785 0458-1786 0459-1787 045A-1788 045B-1789 045C-178A 045D-178B 045E-178C 045F-178D 0460-178E 0461-178F 0462-1790 0463-1791 0464-1792 0465-1793 0466-1794

symaudit -sid 417 monitor -v -text -i 5

Release Sym locks

symcfg -sid 626 -lockn all list

symcfg -sid 626 release -lockn 15

Enable SCSI-3 persistent binding on dewices

Run symconfigure using folowing procedure

set dev 0D53 attribute=SCSI3_persist_reserv;

set dev 0D5B attribute=SCSI3_persist_reserv;

set dev 0D63 attribute=SCSI3_persist_reserv

Volume Set Addressing for HP-UX

Trying to set up FA port for attachment to HP host. This requires VOLS port flag enabled (also known as V-bit, VSA or Volume Set Addressing). Volume Set Addressing limits LUNs to 0-7 only. Targets 0-F and vbus 0-F values can also be used to extend addressing.  The FA port had existing devices configured with LUNs which were not in this range.

An address of 123 represents: vbus=1, target=2, LUN=3 (this would be a valid LUN with VOLS flag enabled).   Note, the overall address of a device is commonly referred to as its LUN, and this includes all three values above.

Some examples to help clarify what are acceptable with the VOLS bit set;

set port 8b:1 volume_set_addressing=enable;

Sym Commands

symdev   Performs operations on a device given the Symmetrix device name

sympd   Performs operations on devices given the devices physical name (c2t0d0)

symgate  Performs operations on gatekeeper devices

symdg   Performs operations on Symmetrix device groups

symld   Performs operations on devices within a device group

symbcv   Performs support operations on BCV pairs

symmir   Performs control operations on BCV pairs

symrdf   Performs control operations on RDF pairs

symcfg discover Creates a local database of the attached symmetricies

syminq    Shows internal & external devices that the host sees

## Power Path

powermt Commands:

powermt check

powercf -q

powermt restore

powermt check (answer to Clear the dead devices)

powercf -q (removes logical links)

devfsadm -C (scans for new devices, removes missing devices)

powermt config (this will restore all paths)

powermt display dev=all ( display paths)

vxdctl enable (rescan for veritas to use)

PowerPath

Install powerpath;

cd /usr/emc

pkgadd -d .

Add registration keys:

# emcpreg -add BVPA-3B47-YFAW-Q9MQ-MC9F-2M6W

1 key(s) successfully added.

cat /etc/emcp_registration

# DO NOT EDIT!  EMC PowerPath Registration data. VER 100

BVPA-3B47-YFAW-Q9MQ-MC9F-2M6W

#powermt check_registration

Check powerpath policy is set to SymOpt

powermt display dev=all

If not set ,set to SymOpt or Clariion Opt.

powermt set policy=so

powermt set policy=co

powermt set policy=co dev=all

powermt save

Remove powerpath :

rm /etc/powermt.custom

rm /etc/powermt.custom.save

rm /kernel/drv/emcp.conf

rm -r /opt/EMCpower

pkgrm  EMCpower