Global Naming Conventions (SAN & Mainframe)

Approval List:

| Title: | Name: | Signature: | Date: |
| --- | --- | --- | --- |
| Document Owner | Scott Higgins |  |  |
| Approver | John Dybus |  |  |
|  | Afzal Mahmood |  |  |
|  | Doug Chin |  |  |
|  | Raymond Borromeo |  |  |
|  | Claire Carpinelli |  |  |

Distribution List

To all SAN & Mainframe Management Employees.

Note:  If printed, content is valid only for the day it was printed (7/15/2015 1:30 PM).  Always refer to the on-line document for the current version.  If the user elects to maintain copies of this procedure, it is their responsibility to verify the currency of the document by checking it against the online version.  This document must be promptly removed from use when obsolete.

1.	Purpose	4

2.	Revision history	5

3.	Persons affected	7

4.	Policy	7

5.	Definitions	7

6.	Responsibilities	8

7.	Assumptions	8

8.	Conventions	9

8.1	Storage Array Naming Convention	9

8.2	Switch Naming Convention	10

8.3	SAN Server (non-SOE & SOE) Naming Conventions	11

8.4	SAN Naming Convention	12

8.5	Fabric SAN Naming Convention	12

8.6	Fabric Active Zone Set Naming Convention	13

8.7	Fabric Zone Set Backup Naming Convention	13

8.8	Fabric Disk Zone Naming Convention	14

8.9	Fabric Tape Zone Naming Convention	14

8.10	WWPN Host Alias (Nickname) Naming Convention	15

8.11	WWPN Disk Storage Alias Naming Convention	15

8.12	WWPN Tape Alias Naming Convention	16

8.13	Hitachi Host Group Naming Convention	16

8.14	Hitachi Host group WWPN Alias Naming Convention	16

8.15	HDP Pool Naming & Numbering	17

8.16	Volume Control Manager Data Base (VCMDB) ASCII WWN Naming Convention	17

8.17	Cisco Domain VSANs	18

8.17.1	Southwest – SAN 3 (usucd03)	18

8.17.2	Midwest – SAN1 (usmcd01)	18

8.18	Cisco Startup/Running Configuration	18

8.19	Cisco Backups	18

8.20	Symmconfigure Command File Naming Convention	18

8.21	Symmask Command File Naming Convention	18

8.22	Symmask Backup File Naming Convention	19

8.23	Symmask List File Naming Convention	19

8.24	VCM Backups	19

8.25	SYMACL Backups	19

8.26	SANScreen Plan Naming Convention	19

8.27	SAN Control Workstations	19

8.28	SANScreen Tape Identification	19

8.29	SAN Control Workstation Naming Convention	20

8.30	VMAX Masking Views	20

8.31	VMAX Initiator Group	20

8.32	VMAX Storage Group	20

8.33	VMAX Port Group	20

8.34	VMAX Pool Names	21

8.35	Symmetrix Device Group Names	21

9.	Appendix A	23

9.1	Country & Site Code IDs:	23

9.2	Approvals	24

# Purpose

This document establishes naming conventions for components and related entities within the Storage Area Network (SAN) and Mainframe environments.

# Revision history

| Date | Rev. No. | Change | Ref. Section(s) |
| --- | --- | --- | --- |
| 06/13/2005 | 1.0 | New Procedure Drafted | Not Applicable |
| 06/17/05 | 1.1 | Added B=Both in section 7.2 Switch Hardware Identifier. Added new section (7.3) for Service Processor (SP) naming convention. | 7.2, 7.3 |
| 06/17/05 | 1.2 | Added SP to list of definitions. Updated document header, and Table of Contents (TOC) | 5.0, |
| 06/23/05 | 1.3 | Change site codes as follows: 1. Changed 111 Wall from “A” to “C” 2. Changed Amherst from “C” to “A” 3. Deleted Code “P=Plainview” | Appendix A |
| 10/26/2005 | 1.5 | Updated san zoning naming convention. | 7.5 |
| 10/26/2005 | 1.6 | Updated tape san zoning naming convention. | 7.6 |
| 03/24/2006 | 1.7, 1.8 | - Changed “Logical” fabric identifier to “SAN” identifier. - Changed SAN identifier from letters A thru Z to numbers 00 thru 99. - Changed “Physical” fabric identifier from numbers 0 thru 9 to letters a or b. - Changed zone names. | 7.2, 7.3, 7.5 |
| 5/5/2006 | 1.9 | - Created Zone Set naming convention. - Updated Disk zone naming convention. - Updated Tape zone naming convention. - Inserted new Assumptions section. - All sections renumbered. - Deleted Appendix B. - Deleted Zone Member naming convention. | 6.0 thru 9.0 |
| 5/19/2006 | 1.10 | - Developed new alias naming convention for host WWPN’s. | 8.8 |
| 6/9/2006 | 1.11 | - Developed new convention for Disk storage port WWPN aliases. - Developed new convention for Hitachi host group naming. - Developed new convention for Hitachi host group WWPN aliases. - Developed new convention for EMC VCMDB ASCII WWN’s. | 8.9, 8.11, 8.12, 8.13 |
| 07/03/2007 | 1.12 | Change site codes as follows: - Deleted Code “G=Gaithersburg” - Added Code “G=Georgetown” (GTDC-TX) - Deleted Code “M=Stamford” - Added Code “M=Midwest” (MWDC-OH) - Renamed “U=Southwest” to “U=Roanoke” (SWDC-TX) | Appendix A |
| 9/4/2007 | 1.13 | Add Names for Cisco Domain ID’s/VSANs |  |
| 6/12/2008 | 1.14 | Added a section for Fabric Names Updated Section for SAN Names to distinguish between Vendors Add site codes for O’Fallon, Frederick, and Pittsford Updated Logo Updated Document Approvers | Appendix A |
| 3/13/2009 | 1.15 | Updated name of the document and changed ownership. Added Section for HDP Pools, SANScreen Tape Identifiers, and Cisco Backups | All |
| 11/17/2009 | 1.16 | Added VMAX entries. Updated site codes to include NAIT. Added approval section. | 9.1, 9.2, 8.30->8.34 |
| 1/4/2010 | 1/17 | Updated document to include the naming of SAN control workstations and zoning/alias names for Cisco/McData merged fabrics. |  |
| 2/1/2010 | 1.18 | Removed Cisco/McData alias change |  |
| 11/7/2011 | 1.19 | Added Country code to all naming standards | 8.1 thru 8.7, 8.15, 8.17, 8.27, 8.29 and Appendix A |
| 1/13/2011 | 1.20 | Added all Mainframe site info and names | Sections 8 & 9 |
| 9/10/2013 | 1.21 | Add M – San Marcos in El Salvador & review | Appendix A |
| 9/30/2013 | 1.22 | Removed I – Information Park from SG (Singapore); Added I – Information Park to CN (China) Added T – Telepark, Tampines to SG (Singapore) | 9.1 |
| 6/2/2014 | 1.23 | Added CENESP Sao Paolo, Brazil Added Citi Center Sao Paolo, Brazil | 9.1 |
| 10/1/2014 | 1.23 | Added Campus, Guatemala (GT) Added Centrica, Guatamala (GT) Added Penata, Guatemala (GT) Added Chocavento, Peru (PE) Added Camino Real, Peru (PE) | 9.1 |
| 10/1/2014 | 1.23 | Updated Switch Naming Standard to include Storage Edge as an identifier and 3-digit sequence number. | 8.2 |
| 3/6/2015 | 1.24 | Added Petapa, Guatemala (GT) Added Kio, Guatamala (GT) Added Santa Elena, El Salvador (SV) Added Palic, El Salvador (SV) Added Plaza Tempo, Costa Rica (CR) Added TerraCampus,Costa Rica (CR) Torres de la Americas, Panama (PA) Panama,Pacifico, Panama (PA) | 9.1 |
| 4/28/15 | 1.25 | Added TIGO Fraijanes, Guatemala (GT). It’s a 3rd party company hosting CoB. | 9.1 |
| 5/6/15 | 1.26 | Updated Panama Pacifico to Lost Pueblos to reflect replaced Tech Room | 8.2 |

# Persons affected

These procedures apply to Data Center Hardware / Service Delivery Teams, Distributed Storage Management (DSM), Distributed Storage Integration & Reporting (DSIR) and any other entities involved in naming Storage Area Network (SAN) or Mainframe components.

# Policy

The policy of GSM is to ensure:

Storage Area Network (SAN) & Mainframe components are named and referred to, in appropriate documentation by a common and agreed upon naming standard.  By doing so, the risk of errors related to misrepresented equipment and components can be mitigated.

# Definitions

- SAN – Storage Area Network.

- HBA – Host Bus Adapter

- GK – Gatekeeper device which is used to communicate SCSI commands from the Control workstation to the EMC Symmetrix storage array.

- BIN – The configuration file which is applied to, or obtained from the EMC Symmetrix storage array.

- VCMDB – The LUN masking security database which resides within the EMC Symmetrix storage array.

- FA – Fibre Channel Adapter.  The host side interface for connecting to the EMC Symmetrix storage array.

- FICON – Mainframe Fibre Channel interface to the EMC Symmetrix storage array.

- ISL – Inter-Switch Link.  Used to connect 2 or more Fibre Channel directors or switches together so they form a network or “Fabric”.

- WWPN – World Wide Port Number.  A unique identifier used in the SAN environment to identify a fibre channel initiator or target.

- SYMCLI – EMC’s Solutions Enabler Command Line Interface.

- CM – EMC’s Connectrix Manager Graphical User Interface.

- Zone Set – A collection of SAN Zones which are activated as a whole group in a SAN.

- Zone – A collection of Zone members comprised of WWPN identifiers which permit communication between WWPN members.

- FTL – Floor Tile Location.

- SP – Service Processor:  A vendor supplied (usually) Computer or Laptop which is used to directly interact with hardware, sometimes using proprietary software and/or applications.

# Responsibilities

These standards and conventions are the shared responsibility of the Distributed Storage Integration & Reporting (DSIR), Distributed Storage Management (DSM) and Data Center Operations teams.

# Assumptions

- Once new naming conventions are proposed and accepted by meeting participants, there will be 7 days of review.  After the 7th day of review, and absent any legitimate reason to stay ratification of the convention, all new SAN & MF entities will follow the new naming convention and plans will be drawn to rename old entities.

- All Fabric Zone sets, Zone names, zone aliases, nicknames, etc. will be lowercase.

# Conventions

## Storage Array Naming Convention

This convention is for all storage arrays currently supported in our datacenters.  These should be used as both physical identification and logical identification.

Array Naming Standards

Country Code ( <Country> See Appendix A)

Mode Identifier; O = Open Systems, M = Mainframe

Site Identifier ( <Site> See Appendix A)

Vendor/Hardware Identifier (See below)

Three digit seq. number (Replicated frames must match)

Replication Indicator: R = Replicated; N = Non-Replicated

Last Four digits of storage array serial number

CC M S V NNN R NNNN

Vendor/Hardware Identifier:

E = EMC Enterprise array (Symmetrix, DMX, VMAX)

C = EMC Clariion, VNX array

H = Hitachi Enterprise array (VSP, USPV, USPVM)

I = IBM Enterprise array (XiV, ESS)

T = EMC Centera array

V = Virtual Tape Library

## Switch Naming Convention

This convention is for all SAN/MF fibre switches/directors currently supported in CTI datacenters.  These should be used as both physical identification and logical identification.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

SAN Fabric Identifier; A or B side

Usage Identifier; S=Storage Edge C=Core, E=Edge

SAN Member Sequence Number (01-999)

CC S V H NN F U NNN

Vendor Identifier:

B = Brocade

C = Cisco

M=McData

## SAN Server (non-SOE & SOE) Naming Conventions

This convention is for all SAN/MF (non-SOE) vendor supplied servers (SVP’s) in CTI datacenters.  These should be used as both physical identification and logical identification.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

SAN Fabric Identifier; A or B side

Always SP for Service Processor

Service Processor Sequence Number (1-9)

CC S V H NN P SP N

Vendor Identifier:

B = Brocade

C = Cisco

I = InRange

M = McData

This convention is for all SAN/MF Management (SOE) servers in CTI datacenters.  These should be used as both physical identification and logical identification.

Network Domain (3 letters, NAM, LAC, EUR, etc.)

Site Identifier ( <Site> See Appendix A)

Short freeform description of server purpose (Cisco, HDS, CWS, etc.)

SAN Fabric Identifier; A or B side (only use if A-B SAN support needed)

SAN Server Sequence Number (01-99)

NNN S DDDDD P NN

## SAN Naming Convention

This convention is for naming the physical SAN within the data center.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

CC S V H NN

Examples:

usucd01

mxcmd01

## Fabric SAN Naming Convention

This convention is for naming the fabric within the data center.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

SAN Fabric Identifier; A or B side

CC S V H NN P

Examples:

usucd01a

mxcmd01a

## Fabric Active Zone Set Naming Convention

This convention is for naming the zone set that is both active in the physical fibre switches / directors and the zone set stored in the service processor zone library.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

SAN Fabric Identifier; A or B side

Constant “-active”

CC S V H NN F -active

Vendor Identifier:

B = Brocade

C = Cisco

I = InRange

M = McData

Examples:

usucd01a-active

mxcmd01a-active

## Fabric Zone Set Backup Naming Convention

This convention is for naming the Zone set copies that are used as backups of the active zone set stored in the service processor zone library.

Country Code ( <Country> See Appendix A)

Site Identifier ( <Site> See Appendix A)

Vendor Identifier (see below)

SAN Usage Identifier; D=Disk, T=Tape, B=Both

SAN Number Identifier (01-99)

SAN Fabric Identifier; A or B side

Constant -"-b" followed by timestamp (YYYYMMDDHHMM)

CC S V H NN F -bYYYYMMDDHHMM

Vendor Identifier:

B = Brocade

C = Cisco

I = InRange

M = McData

Examples:

usucd01a-b201107151815

mxcmd01a--b201111072030

## Fabric Disk Zone Naming Convention

Hostname-HBinstanceID-fabricID

Where:	Hostname = Server Name.

HB       = 2 character constant to indicate HBA.

Instance = unique HBA instance number (0, 1, etc.)

ID       = single character fabric Identifier.

Examples

aiasnyc1-hb0-a

aiasnyc1-hb2-b

easps5-phys-hb0-a

easps5-phys-hb1-b

## Fabric Tape Zone Naming Convention

Hostname-HBinstance-tapegroup-fabricID

Where:  Hostname   = Server Name.

HBInstance = 2 character constant “hb” followed by unique

vendor specific instance number (hb0, hb1, etc.)

tapegroup  = Name of tape pool.

fabricID   = single character fabric Identifier.

Examples:

aiasnyc1-hb0-tapegroup1-a

aiasnyc1-hb2-tapegroup1-b

easps5-phys-hb0-tapegroup1-a

easps5-phys-hb1-tapegroup1-b

## WWPN Host Alias (Nickname) Naming Convention

This convention is for naming the alias of the host WWPN(s) (a.k.a. Nicknames) for both tape and disk connections in the SAN switches/directors.

Hostname-HBinstanceID-fabricID

Where:	Hostname = Server Name.

HB       = 2 character constant “HB” to indicate HBA.

Instance = unique HBA instance number (0, 1, etc.)

ID       = single character fabric Identifier.

Examples

aiasnyc1-hb0-a

aiasnyc1-hb2-b

easps5-phys-hb0-a

easps5-phys-hb1-b

## WWPN Disk Storage Alias Naming Convention

This convention is for naming the alias of the Storage array WWPN(s) connections in the SAN switches/directors.  This includes FA’s on EMC arrays and CL’s on HDS arrays.

Storagearrayname-FAname

Where:	Storagearrayname = Array name based on the storage array naming convention.

Portname = FA/CL Identifier.

Examples

oeh059n1118-cl1e

oue003n0251-fa07aa

## WWPN Tape Alias Naming Convention

Convention to be developed here...

## Hitachi Host Group Naming Convention

This convention is for naming host groups within the Hitachi storage array.  One host group per host will be created following this naming convention.  For cluster hosts or host sharing, individual host groups will still be created.

Hostname

Where:	Hostname = Server/host name.

Examples

crdap1c

dnamd38sql1

## Hitachi Host group WWPN Alias Naming Convention

This convention is for naming the alias of the host WWPN(s) within the Hitachi storage array host group(s).

Hostname-HBinstanceID-fabricID

Where:	Hostname = Server Name.

HB       = 2 character constant “hb” to indicate HBA.

Instance = unique HBA instance number (0, 1, etc.)

ID       = single character fabric Identifier.

Examples

crdap1c-hb0-a

crdap1c-hb2-b

dnamd38sql1-hb0-a

dnamd38sql1-hb1-b

## HDP Pool Naming & Numbering

HDP stands for  Hitachi Dynamic Provisioning (aka Thin Provisioning).

HDP pools are named as follows:

NNNN_Size (pool number)

NNNN=The last 4 digits of the Hitachi frame serial number (VSP on only)

Size=Size of drives in the pool. 300GB, 450GB, 2TBSATA etc. (VSP on only)

Pool number=Pool number (starting with 20 to match thin starting CULDEV numbers)

Examples:

5592_300GB (20)   
5592_2TBSATA (21)

Please refer to HDS Solutions guide published by storage engineering for details.

## Volume Control Manager Data Base (VCMDB) ASCII WWN Naming Convention

This convention is for naming the ASCII WWN of the host WWPN(s) within the EMC Volume Control Manager Database (VCMDB).

Hostname/HBinstanceID-fabricID

Where:	Hostname/ = Server Name followed by the constant “/”.

HB        = 2 character constant “hb” to indicate HBA.

Instance  = unique HBA instance number (0, 1, etc.)

ID        = single character fabric Identifier.

Examples

crdap1c/hb0-a

crdap1c/hb2-b

dnamd38sql1/hb0-a

dnamd38sql1/hb1-b

## Cisco Domain VSANs

This convention follows the San Fabric naming described previously.

Examples

### Southwest – SAN 3 (usucd03)

| VSAN ID | VSAN Names | Reserve Domain IDs |
| --- | --- | --- |
| 1 | Default | N/A |
| 2 | usucd03a |  |
| 3 | usucd03b |  |

### Midwest – SAN1 (usmcd01)

| VSAN ID | VSAN Names | Reserve Domain IDs |
| --- | --- | --- |
| 1 | Default | N/A |
| 14 | usmcd01a |  |
| 15 | usmcd01b |  |

## Cisco Startup/Running Configuration

switch_running_config_timestamp

Where:	Switch       = Name of Physical Switch

Config Type	= Running or Starting

Starting

Config	= Config

Timestamp 	= YYYYMMDDHHMM (Military Time)

Example

ucd01ac01_starting_config_200709181807

## Cisco Backups

Convention to be developed here...

## Symmconfigure Command File Naming Convention

Convention to be developed here...

## Symmask Command File Naming Convention

Convention to be developed here...

## Symmask Backup File Naming Convention

Convention to be developed here...

## Symmask List File Naming Convention

Convention to be developed here...

## VCM Backups

Convention to be developed here...

## SYMACL Backups

Convention to be developed here...

## SANScreen Plan Naming Convention

This convention is for a standard way in which to name SANscreen plans.

Hostname-target date-serial number-FAA-FAB

Where:

Hostname – name of server

Target Date – target date for execution of the work.

Serial number – Serial number of the storage array.

FAA – Fabric A FA/CA/SP

FAB – Fabric B FA/CA/SP

For example

namuemc01-20060623-0001-8AA-9BA

## SAN Control Workstations

See section 8.3 for all standard non-SOE and SOE server naming standards.

Sample names:

namucws01 – US network domain (NAM), U=SWDC, CWS (description), 01- first server

lacpcws01 – Latin American network domain (LAC), P=Jardines, CWS (description), 01- first server

## SANScreen Tape Identification

Convention to be developed here...

## SAN Control Workstation Naming Convention

See section 8.3 for all standard non-SOE and SOE server naming standards.

## VMAX Masking Views

The naming convention will be “hostname”_mv.  If there are more than one masking view, it will be “hostname”_mv”x”, where X is 1->……

For example:

sanccw02_mv

sanccw02_mv1

sanccw02_mv2

## VMAX Initiator Group

The naming convention will be “hostname”_ig.  If there are more than one masking view, it will be “hostname”_ig”x”, where X is 1->……

For example:

sanccw02_ig

sanccw02_ig1

sanccw02_ig2

## VMAX Storage Group

The naming convention will be “hostname”_sg.  If there are more than one masking view, it will be “hostname”_sg”x”, where X is 1->……

For example:

sanccw02_sg

sanccw02_sg1

sanccw02_sg2

## VMAX Port Group

The port groups will be named with the FA addresses plus a pg at the end.

For example:

fa07ea_08ea_pg

fa07eb_08eb_pg

fa07ea_08ea_07eb_08eb

## VMAX Pool Names

The VMAX Pool Names will be as follows:

“Last Four Digits of Serial Number” Drive Size

For example

0260_600GB

## Symmetrix Device Group Names

The device group names should be as follows:

hostname_device type

Device Type:

SRDF – SRDF R1

BCV – R2 that has a BCV associated with it.

R2 – R2 with no BCV associated with it.

Sancw01_SRDF

Sancw01_BCV

Sancw01_R2

If the host is part of a cluster, it should note the two pieces of the name that are different.

For example,

Sancw01-1, sancw01-2

Sancw01_2_SRDF

Sancw01-phys, sancw01-bkup

Should be names

Sancw01_phys_bkup_SRDF

If the DG name needs to account for both local and global zones, the name should be:

Localzone_globalzone_device type..

For example:

R1 DG for

Local Zone – sancw01

Global Zone – sans

Sancw01_sans_SRDF

# Appendix A

## Country & Site Code IDs:

<Country>

Use the internet ISO 3166 standards (see Wiki link for details).

http://en.wikipedia.org/wiki/ISO_3166-1_alpha-2

<Site>

City or site description

US (United States)

- A- Albuquerque

- D - Des Moines

- E - 390G (390 Greenwich Street, New York, NY 10013)

- G - GTDC (201 Blue Ridge Drive, Georgetown, TX 78626)

- H - Regent

- I - Irving

- J – Jacksonville (14000 Citi Cards Way, Bld C, Jacksonville, FL 32258)

- K - Kansas City

- M - MWDC (Midwest, 306 Grief Parkway, Delaware, OH 43015)

- N – Nevada (8725 W. Sahara Ave, Las Vegas, NV, 89163)

- O - O'Fallon (1000 Technology Drive)

- R – Rutherford (700 Edwin L. Ward, Sr. Memorial, Rutherford, NJ 07070)

- S - SSMD (12401 Prosperity Drive, Silver Spring, MD 20904)

- T – Tampa (3800 Citigroup Center, Tampa, FL 33610)

- U - SWDC (Southwest, 931 Litsey Road, Roanoke, TX 76262)

- X - Sioux Falls

- Y - Dallas

- Z - Columbus

AR (Argentina)

- B – Bartolome Mitre

- V- Villa Adelina

BR (Brazil)

- B – Av Maria Coelho I

- C – Sao Paulo (CEAB Centro Empresearil Agua Branca)

- S – Sao Paulo (CITI Center SP)

- W – Sao Paulo (CENESP)

BS (Bahamas)

- O – Oaks Field (CITI Trust Bahamas Limited, 110 Thompson Blvd, Nassau, Bahamas)

CA (Canada)

- B – Mississauga (5900 Hurontario St, Mississauga, Ontario; also includes 2920 Matheson)

- L- London

- T – Toronto (123 Front St, Toronto, Ontario)

CN (China)

- C - #1 Jian Guo Men Nei Ave

- I – Information Park

CO (Columbia)

- B – Bogota

CR (Costa Rica)

- P – Plaza Tempo

- T – TerraCampus

DE (Germany)

- D – Dusseldorfer Strasse 71B

- L – Heinrich-Lanz-Allee 47

GB (United Kingdom)

- K – 66 Molesworth Street

- Q – 33 Canada Square

GT (Guatemala)

- C – Campus

- N –Centrica

- K - Kio

- P – Petapa

- T – TIGO Fraijanes (3rd Party company hosting CoB)

HK (Hong Kong)

- H – 399 Chai Wan Road

- A – 1 Wang Wo Tsai Street

KR (South Korea)

- T – 1127 Kuwol-Dong, Namong-Gu

- V – San 1-8 Mabuk-Ri, Gusung-Eub

MX (Mexico)

- C – Monterrey (Monterrey, Nuevo Leon)

- F – Santa Fe (Mexico City, Mexico)

- P – Jardines (Pico De Verapaz #331, Mexico City, Mexico)

- Q - Queretaro (QRDC)

PA (Panama)

- T – Torres De la Americas (TDA)

- P - Panama Los Pueblos

PE (Peru)

- C – Chocavento

- R – Camino Real

SG (Singapore)

- N – 75 Science Park Drive

- T – Telepark, Tampines Center

SV (El Salvador)

- E – Santa Elena

- M – San Marcos (GFU - EDIFICIO SAN MARCOS, LOTE 5D-A KM. 7 ½, AUTOPISTA A COMALAPA, SAN MARCOS, SAN SALVADOR, EL SALVADOR)

- P -Palic

- S - Santa Tecla

VE (Venezuela)

- C -Caracas