# Key storage performance metrics for virtual environments

Chris Evans

- A

- AA

- AAA

- Print

- Email

- Reprints

- Facebook

- LinkedIninShare37

- Share This

- Tweet

Delivering efficient and optimised storage performance for virtual environments can be difficult to achieve because so many components in the infrastructure stack must interact.

But optimum storage performance can be achieved with a starting point that begins with a set of metrics to measure storage operations in the virtual environment.

Getty Images/iStockphoto

There are three key storage performance metrics common to all environments: Latency, throughput (or bandwidth) and IOPS.

Latency (also called response time) describes the time taken to complete a single I/O (input output) operation and is essentially a measure of how fast a storage system responds to read and write requests.

Values are usually measured in milliseconds, with the fastest flash drives now quoting fraction of a millisecond figures. In an ideal world, latency would be zero, resulting in no penalty to the application for read/write operations to permanent storage media. But, physics gets in the way and some latency exists for every I/O operation.

The aim for any storage solution is to minimise latency values, as storage is often the bottleneck in IT infrastructure. Lower latency means less waiting for the completion of I/O and therefore results in faster execution.

In virtual environments latency has a direct impact on the speed at which virtual machines (VMs) and desktops operate and reduced latency means more efficient use of processor and memory.

As a result, we have seen the adoption of solid-state storage into virtual environments and the movement of I/O management into the server. Here flash-caching hardware and software aims to eliminate the need for data to traverse the network, thereby producing very low latency values.

Throughput – The capability of a storage system to transfer a fixed amount of data in a measured time is known as throughput, or bandwidth. Typically, throughput is measured in megabytes per second (MBps) or similar units.

Storage arrays and disk devices can be measured for two throughput metrics - sustained throughput and peak throughput. Sustained throughput is a measure of the constant capability of a device or system over a long period of time. Peak throughput indicates the levels a system can provide over short periods.

Peak throughput levels are important in VDI (virtual desktop infrastructure) environments, where boot storms - when many users log into the system and start up their virtual desktops at the same time - can generate huge I/O demand, resulting in poor performance and a rise in latency if the system cannot manage the spike effectively.

Good throughput numbers are also essential for virtual server environments when managing the dynamic movement of VMs between datastores. Being able to measure throughput and understand peak demand is critical to virtual environments.

IOPS (input output operations per second) – IOPS is a measure of the number of individual read/write requests a storage system can service per second. This figure is closely related but subtly different to throughput.

In many cases, vendors will use IOPS as a measure of the performance of their products, but these figures need to be considered alongside the size of data chunks being transferred in each operation.

For example, many small (say, 4KB) requests are easier to handle than large (1MB) ones. Also reads, especially of random rather than sequential datasets, are generally more time-consuming than writes. So, IOPS claims need to be judged against the volume of data chunks being dealt with and the type of operations they refer to.

#### Read more on storage performance

- Most valuable storage metrics describe performance, capacity

- A Deep Dive into Storage Performance Benchmarks

- Gluttonous VMs can wear down vSphere storage performance

- Five storage configuration tips to boost VM performance

- VDI storage basics and persistent versus non-persistent desktops

### The relationship between latency, throughput and IOPS

Latency, IOPS and throughput are all closely-related. A storage system that can deliver I/O at very low latency will be able to deliver a high IOPS performance. At the most basic level, the number of IOPS achievable is simply 1/latency, so a latency of 3 milliseconds (or 1/0.003) translates to around 333 IOPS.

A system that can deliver a high number of IOPS with large data chunks will be able to deliver a high throughput, as the value is simply the number of IOPS multiplied by the I/O size.

Of course, vendors make claims to very high numbers when quoting performance figures and there are some good reasons for this. Storage arrays are able to manage I/O workload with parallelism or concurrency, handling more than one I/O operation at the same time.

Concurrency is achieved by providing multiple paths to the storage system and using system memory as a cache to queue transactions. This leads us to a new measurement – queue depth – that describes how many I/O requests a device can handle simultaneously.

A single disk drive will have a queue length in single or double figures, whereas a large enterprise array will provide a queue depth into the tens or hundreds per LUN, per port or a combination of both.

By queuing multiple requests together, a storage device can optimise the write process, reducing some of the physical latency of storing data, which is particularly efficient with spinning disk hard drives because head movement can be significantly reduced.

### Workload profiles and where to measure

Identifying and recording metrics provides the raw data for understanding storage performance, but any numbers gained need to be considered in context in terms of I/O profile and where the measurements are taken. This is because all applications produce different workload demands.

As an example, VDI and virtual server traffic are highly randomised due to the dispersal of active data throughout a datastore or volume storing virtual hard disks. VDI data is typically mostly read-heavy (80R/20W as a percentage split or higher); so low read I/O latency gives a significant performance boost.

In the days of the mainframe, a single I/O operation could be tracked from start to finish, showing where delays occurred at each step of the journey. Today, I/O is much more complex, with measurements possible at the storage device, within the hypervisor and within the host itself

Choosing where metrics are recorded is also important to provide an end-to-end view of I/O performance.

In the days of the mainframe, a single I/O operation could be tracked from start to finish, showing where delays occurred at each step of the journey. Today, I/O is much more complex, with measurements possible at the storage device, within the hypervisor and within the host itself.

There’s no right or wrong place to take measurements; each gives a perspective on the operation of the system. Values taken from the array show how well the external storage copes with demand. Values taken from the host, for example, show how contention at the datastore affects individual guest performance while values taken from the hypervisor show the effectiveness of the storage network.

Both the common hypervisors (vSphere ESXi and Hyper-V) allow for the movement of workloads to optimise storage performance. Storage DRS for vSphere, for example, bases VM migration recommendations on historical I/O latency measurements of the underlying datastores. Meanwhile, Intelligent Placement in Hyper-V makes calculations based on VM IOPS.

### Let’s not forget

Most of the numbers we have talked about are technology-based, but there are financial metrics to consider when purchasing products and placing data on tiers of storage. These include $/GB, which measures cost per unit of capacity. But, with the introduction of flash, where $/GB is so much higher than with spinning disk, $/IOPS can have more useful meaning when application performance is a greater consideration.

### Putting it all together

Due to the random nature of virtual environments, latency is a key metric in monitoring the status of physical storage resources. Latency is relevant, whether a system contains one or one hundred VMs.

When we look at the capacity to support many VMs, throughput plays a big role, because the ability to scale a virtual environment requires corresponding capability in throughput. As already discussed, managing peak demand can be an issue for VDI environments which have peak read and write load periods.

From the perspective of the host, IOPS is typically used as the standard measure, because this provides an abstracted view that isn’t dependent on the underlying hardware capabilities. This is seen as a measure in both private and cloud virtual infrastructures.

# Storage performance metrics: How suppliers spin performance specifications

Manek Dubash

- A

- AA

- AAA

- Print

- Email

- Reprints

- Facebook

- LinkedIninShare24

- Share This

- Tweet

Caveat emptor – let the buyer beware – is as true in IT as any walk of life; maybe more.

Organisational buying decisions can be fraught with complexity and have consequences that are far-reaching – and can be career-limiting.

Getty Images/iStockphoto

So when it comes to buying storage products, what should you look out for in suppliers’ storage performance metrics?

Buying storage centres on performance – capacity, latency, throughput and so on – and there are numerous ways manufacturers massage spec sheet figures. But, storage products also come with features, which may or may not be desirable in their own right, but can also affect performance.

So, what are the key ways suppliers mislead customers?

To find out, we asked a range of storage industry analysts to list and explain the ways they see storage suppliers put a spin on their marketing.

### George Crump, Storage Switzerland, president and founder

Do you get the IOPS they say you will?

Buyers need to look at spec sheets in the context of their own environment and compare apples to apples.

For example, Pure Storage, Violin Memory and IBM all publish IOPS benchmarks, but you need to know parameters such as block size and what features were enabled during testing. Features such as data deduplication, compression and thin provisioning affect flash and its low latency in a way that was not an issue with spinning disks.

Beware of benchmarks

Does a benchmark give you enough granular detail to enable you to translate it to your environment? Two tests are valuable – raw IOPS tests and Loginvsi, which fires up a bunch of VDIs and runs them through day-to-day tasks to replicate real world testing.

### Greg Schulz, StorageIO, senior advisory analyst

#### Read more about storage performance

- Storage performance metrics: How to read array supplier specification sheets

- Flash storage 101: How solid state storage works

- Key storage performance metrics for virtual environments

- Top features to boost VMware storage performance

Dodgy performance claims

Startup and mainstream suppliers of disk, flash and hybrid flash storage systems like to toss out performance claims, and sometimes those claims get backed up with an actual metric.

One of the classics is when a supplier says they are 3x (or pick your own number) faster than their most popular competitor and twice as fast as the previous version of their own solution.

What they don’t usually tell you is that they are comparing their newest, optimally configured, lightly loaded system against one of their competitor's oldest and poorly configured systems.

If a supplier makes a claim that they can do “X” number of IOPS or “Y” throughput without any other information, ask them about context

Greg Schulz, StorageIO

Not all IOPS are equal – context matters

Suppliers will say a storage system can do 1 million or 2 million or 8 million IOPS, but you later find out those are 64byte (or smaller) sequential read I/Os that are being cached. This might be real-world for some applications, but out of this world for most others.

So, context matters. If a supplier makes a claim that they can do “X” number of IOPS or “Y” throughput without any other information, ask them about context. Questions should include what size I/Os, whether they are reads, writes or mixed, random or sequential, what was the response time, how much storage space was used and how was it configured?

Are those sales figures for real?

When a supplier says they have shipped 500 systems, is that the total number of instances, or the number of controllers/nodes/processors? For example, if an array maker has a dual controller configuration and says they shipped 500 systems, is that actually 250 arrays with two nodes, or did they ship 1,000 controllers with 500 systems? For cluster, grid and scale-out architectures things get even more interesting, if the supplier has, for example, four, eight, 12 or more nodes in a system.

Thanks for the memory – when it exists

When suppliers quote memory and storage space capacity, they might say a system has 8GB of cache. But, is that 2x4GB if there are two controllers/nodes/processors/systems? And if it’s a dual controller array, is half the cache memory in each node set aside for the partner controller, meaning there is only, for example, 2GB usable per controller?

Some suppliers will also count cache memory for data as well as their own internal memory to show a larger number.

### Tony Lock, Freeform Dynamics, programme director/analyst

The important step is to understand what you really need and try to look at the whole field

Tony Lock, Freeform Dynamics

Don't pay for performance you don't need

I don’t know many organisations that really need or want peak performance. Most customers need products that provide the performance level and characteristics the services supported require. Not less. Not more. Raw performance specs of systems in preconfigured tests are rarely of any use, because test workloads don’t have much in common with the real world.

Do quoted features match your requirements?

Buyers need to accurately identify the features they absolutely have to have, those that might be useful and the rest for which they have no likely use in the foreseeable future. Then they should line this up with architectural requirements to fit into the existing landscape with minimal disruption and their “comfort zones” in terms of skills.

Most storage suppliers have core features that most customers require. Some have detailed technical differentiators that may be important to some clients, but may not be of interest to many others. This is the way IT has always been. The important step is to understand what you really need and try to look at the whole field, not only those suppliers you know well.

Great feature, but do you need it?

Very, very few IT suppliers do a good job of marketing use cases for their solutions. Nearly all are very close to the technology and love talking about bits and bytes, not real-world use. Most rely on their pre-sales teams and channel partners to work these out, although this is changing slowly.

The biggest area where suppliers could improve is to tell customers (and channel partners) not just what technology and software management features their products possess, but to help explain where and why technical features could be important. Few suppliers in storage or any other part of IT do this well.

### Valdis Filks, Gartner research director, 
information technology infrastructures

Suppliers get artificially high performance by avoiding writes to disks and writing to controller cache instead

Valdis Filks, Gartner

Beware of benchmarks - again

Suppliers performance claims generally should be taken with a pinch of salt. Open benchmarks – such as those from the Storage Performance Council (SPC) – are better, but they also need to be read carefully. Generally speaking there is no ideal benchmark, but in terms of transparency the SPC ones are quite useful, because they show the configuration and the price per IOPS.

Some suppliers say benchmarks are useless but then quote benchmarks for some products and not others. This is a good indication of which products do not perform well. The ones without open and transparent benchmarks are the ones to be wary of.

Suppliers cache in

Suppliers get artificially high performance by avoiding writes to disks and writing to controller cache instead. Sure, in reality lots of I/O is cached, but this depends on the workload.

Software smarts are key

Suppliers use the same drives and Intel CPUs in their controllers. This means the one that produces the fastest controller will have a short-lived advantage because, in months, most other suppliers will have the same hardware in their controllers. So, the differentiating factor is the storage software's efficiency.

New storage arrays use memory page management techniques, while older arrays are stuck in Raid groups. Raid group designs do not adapt well to new data reduction techniques, which require higher array performance.

SearchStorage

Top of Form

Search the TechTarget NetworkSearch the TechTarget Network

Bottom of Form

Sign-up now. Start my free, unlimited access.

Login Register

SearchStorage

SearchSolidStateStorage

SearchVirtualStorage

SearchCloudStorage

SearchDisasterRecovery

SearchDataBackup

Topic Enterprise storage, planning and management

Storage Strategy View All

Data center storage

HPC storage

Outsourced storage and storage service providers

Small business storage

Storage management jobs and salaries

Storage market research reports

Storage vendors

Cloud Storage View All

Cloud Computing Infrastructure

Private Cloud

Public Cloud

Data Backup View All

Data reduction and deduplication

Data storage backup tools

Disk-based backup

Tape backup and libraries

Data Protection View All

Disaster recovery and planning

Remote data protection

Secure data storage

Data Storage Management View All

Data management tools

Data storage compliance and archiving

Primary storage capacity optimization

Remote and offsite data storage

Tiered storage

NAS (network attached storage) View All

Clustered NAS

NAS hardware

NAS management

Primary Storage or Storage Hardware View All

Disk arrays

Disk drives

RAID

Solid-State Storage

SAN (storage area network) View All

Fibre Channel (FC) SAN

ISCSI SAN

Multiprotocol or unified storage

SAN (storage area network) management

SAN switch

Storage and Virtualization View All

Data Storage for a Virtual Environment

File Virtualization

SAN Virtualization and Consolidation

Storage Network Virtualization

Virtualization Strategy

Storage Strategy View All

Data center storage

Enterprise storage, planning and management

HPC storage

Outsourced storage and storage service providers

Small business storage

Storage management jobs and salaries

Storage market research reports

Storage vendors

Topics Archives View All

Storage Resources

Please select a category

Cloud Storage

Data Backup

Data Protection

Data Storage Management

NAS (network attached storage)

Primary Storage or Storage Hardware

SAN (storage area network)

Storage and Virtualization

Storage Strategy

Topics Archives

Section Manage

News

Get Started

Evaluate

Manage

Problem Solve

Sponsored Communities

Search the TechTarget Network

Understanding storage performance metrics

0

by: 
Stephen Foskett

Gestalt IT

Learn how to accurately measure storage performance metrics in your data storage system, and discover why your results may differ from vendor benchmark numbers.

In this Article

Product ReviewsPowered by IT Central Station

CA APM

vs.

New Relic

Visual TruView

vs.

AppDynamics

ProactiveNet

vs.

Foglight

Infrared360

vs.

SCOM

Sponsored News

A Deep Dive into Storage Performance Benchmarks –NetApp

New Benchmark Results for Unified Scale-Out Storage –NetApp

See More

Vendor Resources

Measuring Storage Performance to Better Predict Your Storage Needs –CloudByte

A Guide to Storage Efficiency –FileTek, Inc.

Related Content

Understand SPC-1 – SearchStorage

Understanding storage performance metrics – SearchStorage.co.UK

Storage performance monitoring in a sprawling, ... – SearchStorage.co.UK

What you will learn in this tip: Learn how to accurately measure storage performance metrics in your storage system, and discover why your results may differ from vendor benchmark numbers.

Download TechTarget's Guide for IT and Business Managers in Southeast Asia

Access this Guide for IT and Business Managers in Southeast Asia, written by technology experts and advisors, to find out more about cloud migration, big data benefits, and more.

Step 2 of 2:

Top of Form

E-mail Address:

Bottom of Form

By submitting you agree to receive email communications from TechTarget and its partners. Privacy Policy Terms of Use.

Data storage vendors often release benchmark numbers for their systems to give customers an expectation of storage performance levels. But smart customers know not to trust these benchmarks completely because they don't always match real-world performance levels. We'll look at the factors benchmarks measure and why you might get substantially different results than those claimed by the vendor.

Disk throughput, latency and seek time

The most basic storage benchmark is megabytes per second (MBps), which measures disk throughput. As data is moved, performance is gated by the read speed of the source, the write speed of the destination, and the buffers and buses in between.

Manufacturers often report a single performance metric for their products, but real-world results frequently differ. The physical geometry of spinning disks causes data throughput to slow as the heads move from the outer tracks to the innermost tracks. Personal computers used by just one person often have uncontested access to all of these resources, but servers must deal with congestion arising from conflicting application demands. Therefore, you should treat a manufacturer's claimed throughput as a realistic maximum, not a daily expectation. Because they're mechanical devices, the read/write heads of hard disk drives take time to move into position as data is requested. This introduces rotational latency and seek time, and can slow disk performance significantly, especially in busy multitasking environments. Manufacturers generally report an average seek time measured in milliseconds for each model of a hard disk drive, and rotational latency can be calculated based on drive spindle speed. Although not useful on their own, these storage metrics become important when trying to predict storage system performance.

Pro+

Features

Enjoy the benefits of Pro+ membership, learn more and join.

E-Zine

Cloud-based disaster recovery: Just add data for instant recovery

E-Zine

Six hot data storage trends for 2015

E-Zine

The Storage magazine/SearchStorage 2014 Salary Survey

Solid-state drive (SSD) media can dramatically outperform mechanical hard disk drives. Data can be read from any portion of a flash memory drive with virtually no seek time or latency. Read throughput is high as well, but writes are a different story. Because data is stored in large blocks that must be entirely rewritten when even a single bit has changed, throughput and latency of write requests can be unpredictable.

More on storage performance

Troubleshooting and identifying data storage performance bottlenecks 

Enterprise data storage performance monitoring goes beyond storage arrays

Measuring IOPS

Enterprise storage systems incorporate technologies designed to overcome the limitations inherent in hard disk drives, and many familiar storage benchmarks are designed with certain assumptions about these systems. You're likely to encounter two types of benchmarks: synthetic measurements of theoretical performance and real-world application runs.

The grandfather of all storage performance metrics is a simple statement of the number of input/output operations per second (IOPS) that a subsystem can sustain. IOPS can be calculated or measured, and varies significantly based on the size and direction of the operations included. Performance can vary dramatically based on the size of the I/O under measurement, with most systems optimized to handle reads and writes of a few kilobytes.

Writing data to a RAID set or flash drive is often substantially slower than reading that data back. Fortunately, most applications read more than they write. Therefore, most benchmarks include a mix of read and write operations, often skewed toward the former.

We have theoretical models to predict the IOPS an application requires or that can be sustained by a RAID set. The theoretical performance of a given RAID level is often expressed as a write penalty, since parity RAID tends to slow writes more than reads.

Keep in mind that most enterprise systems include cache that accelerates I/O. This tends to upset assumptions based on theoretical IOPS and RAID levels because cache equalizes and accelerates performance.

Standard Performance Evaluation Corporation and synthetic app benchmarks

Because theoretical performance metrics often don't hold up when it comes to real-world systems, many people rely on application workloads for benchmarking. There are two main benefits to using application-based benchmarks: they allow apples-to-apples comparisons of systems with vastly different capabilities, and they suggest real-world storage performance expectations.

For example, a benchmark demonstrating Microsoft Exchange server performance would suggest that customers deploying a similar configuration should experience similar results. Synthetic metrics like IOPS force customers to model their systems and draw conclusions, introducing a greater potential for error.

Yet applications and expectations change frequently, and it's useful to compare the performance of systems separated by years or decades. This has led to the creation of synthetic application benchmarks, with the Standard Performance Evaluation Corporation (SPEC) leading the way. Rather than using actual applications, SPEC benchmarks rely on a mix of I/O intended to simulate a generic application. This balances the need for real-world performance and consistency over time.

Overall, if you seek to understand storage performance metrics, it's important to consider the implications of the questions you ask. Are you looking for real-world application benchmarks, synthetic comparisons over time, or basic measures of throughput and I/O performance? Each can be valid, and can assist in the selection of appropriate technology.

BIO: Stephen Foskett is an active participant in the world of enterprise information technology, currently focusing on enterprise storage and cloud computing. He is responsible for Gestalt IT, a community of independent IT thought leaders, and organizes their Tech Field Day events. A long-time voice in the storage industry, Foskett has authored numerous articles for industry publications and is a frequent presenter at industry events, including Storage Decisions. His contributions to the enterprise IT community have earned him recognition as both a Microsoft MVP and VMware vExpert. He can be found online at GestaltIT.com, FoskettS.net and on Twitter at @SFoskett.

This was first published in October 2010

Dig deeper on Enterprise storage, planning and management

All

News

Get Started

Evaluate

Manage

Problem Solve

Data storage systems: 2014 Products of the Year finalists

How do hardware profiles work in Microsoft Virtual Machine Manager?

Cloud-based disaster recovery: Just add data for instant recovery

What is Microsoft's storage replication feature?

Load More

Data storage systems: 2014 Products of the Year finalists

GE pre-tests storage performance, cuts downtime with Load DynamiX

Six hot data storage trends for 2015

HP 3PAR StoreServ platform grows with new hybrid array

Load More

How do hardware profiles work in Microsoft Virtual Machine Manager?

Cloud-based disaster recovery: Just add data for instant recovery

file-area network (FAN)

Non-disruptive upgrade (NDU)

Load More

Data storage systems: 2014 Products of the Year finalists

Cloud-based disaster recovery: Just add data for instant recovery

What is Microsoft's storage replication feature?

Six hot data storage trends for 2015

Load More

How do hardware profiles work in Microsoft Virtual Machine Manager?

Cloud-based disaster recovery: Just add data for instant recovery

What is Microsoft's storage replication feature?

What are the most obvious object storage use cases for my system?

Load More

Six hot data storage trends for 2015

How do I increase cache memory?

Use third-party SRM software to improve monitoring and management

Effective storage service management rests on well-allocated features

Load More

0 comments

Oldest  Newest

Top of Form

Bottom of Form

Top of Form

Register or Login

E-Mail

Username / Password

Password

Forgot your password?

By submitting you agree to receive email from TechTarget and its partners. If you reside outside of the United States, you consent to having your personal data transferred to and processed in the United States. Privacy

Bottom of Form

Load More Comments

Forgot Password?

No problem! Submit your e-mail address below. We'll send you an email containing your password.

Top of Form

E-Mail:

Submit

Bottom of Form

Your password has been sent to:

-ADS BY GOOGLE

Latest TechTarget resources

Solid State Storage

Virtual Storage

Cloud Storage

Disaster Recovery

Data Backup

SearchSolidStateStorage

Dell ships cheaper entry-level all-flash storage arrays

Dell ships two entry-level all-flash storage arrays with advertised street prices of $25,000 and $35,000 to address customer ...

OCZ, Lite-On show off new enterprise M.2 SSDs at CES

New enterprise M.2 SSDs from OCZ, Lite-On appeal to hyperscale companies and general enterprise IT for booting servers, caching ...

NexGen Storage to market ioControl array following SanDisk spinout

NexGen Storage said it's revamped the hybrid storage array and will release product details later this month after getting spun ...

SearchVirtualStorage

Citrix WorkspacePod reference architecture uses Sanbolic, Moonshot

WorkspacePod reference architecture pairs HP hardware with Sanbolic software-defined storage that Citrix acquired this week.

Citrix buys startup Sanbolic, steps into hyper-converged market

Desktop virtualization firm Citrix acquired software startup Sanbolic to create a new storage division and compete in ...

Storage system software: 2014 Products of the Year finalists

Vendors that describe their products as ‘software-defined storage’ dominated the 2014 Products of the Year storage system ...

SearchCloudStorage

Cloudian adds Hadoop to HyperStore for big data analytics

Cloudian enhances its HyperStore software with Hadoop integration and geo-replication for hybrid cloud storage as it tries to ...

Build an OpenStack storage cloud

OpenStack's storage components -- Cinder and Swift -- let you build block and object storage systems in your private cloud. Chris...

Deep dive into cloud-based DR

Called cloud's killer app, cloud-based DR puts fast, economical recoveries within reach of most companies.

SearchDisasterRecovery

Sanovi plants a stake in the disaster recovery services market

Sanovi has customized its application-aware disaster continuity and recovery software for public, private or hybrid clouds.

Using ANSI/ASIS SPC.2-2014 for auditing risk-based management systems

Paul Kirvan looks at ANSI/ASIS SPC.2-2014, a standard designed to facilitate audits of management systems that focus on "risk, ...

How to use FINRA Rule 4370 for business continuity management

If you're not familiar with FINRA Rule 4370, Paul Kirvan explains why you should know about this emergency preparedness rule for ...

SearchDataBackup

Sphere 3D adds 3 TB drives to Overland's RDX QuikStor backup appliance

Sphere 3D promises 3 TB RDX removable drives in its new QuikStor backup appliance, acquired in its $81 million merger with ...

CommVault, NetApp partner on integrated backup appliance

CommVault Backup Appliance is sold on NetApp E-Series hardware; backup software vendor looks to take on Symantec, EMC appliances.

Asigra Cloud Backup will add Docker support in next version

Asigra will add Docker support for its container backup and recovery in version 13. Although early adoption rates may be limited...

About Us

Contact Us

Privacy Policy

Videos

Photo Stories

Guides

Advertisers

Business Partners

Media Kit

Corporate Site

Experts

Reprints

Archive

Site Map

Events

E-Products

All Rights Reserved, copyright 2000 - 2015, TechTarget

In this Article

Sponsored News

A Deep Dive into Storage Performance Benchmarks –NetApp

New Benchmark Results for Unified Scale-Out Storage –NetApp

See More

Vendor Resources

Measuring Storage Performance to Better Predict Your Storage Needs –CloudByte

A Guide to Storage Efficiency –FileTek, Inc.

Close